# Lecture 02 - Signals, Systems, and Transforms

EE-30023/31023, Department of Electrical Engineering, University of Notre Dame

## Summary

In this lecture, we will review some of the important concepts and terminology, and our notation, from what is commonly called signals and systems.

We focus initially on single-input, single-output, linear time-invariant (LTI) systems. The key idea in LTI systems is that their effect on complex exponential signals is particularly simple. This motivates transforms between the time domain and frequency domain through we represent general signals as linear combinations of complex exponentials.

We address these notions in both continuous-time domain and discrete-time domain, and set the stage for connecting the two domains in the next lecture.

## Motivation in Course Context

Three important components for modeling the radio system architectures that we discussed in the previous lecture are 1) an digital-to-analog converter (DAC) in the transmitter, 2) a propagation “channel” that we aim to model as a linear, time-varying filter, and 3) the analog-to-digital converter (ADC) in the receiver. A block diagram for such a system model is shown in Figure 1.

Figure 1: Radio transmission system model

In this lecture, we review the background material required to treat these inputs and outputs as continuous-time signals and discrete-time sequences, and we develop the appropriate notions of the frequency domain and the important class of linear time-invariant (LTI) systems. In our next lecture, we will address sampling, interpolation, and quantization aspects of DACs and ADCs.

## Signals and Sequences

When we wire up an electrical circuit, we can use an instrument to measure a voltage or current, either of which typically varies over time.

Mathematically, we denote time by the real variable t∈R, which we refer to as the continunous-time axis, and we denote the voltage or current by the real-valued functions vt and it, respectively.

More generally, we can consider any quantity that varies as a function of time as a signal, and we generically denote it by st. As other examples, we could measure the temperature in a room, or the price of a stock, or … as functions time.

If, for whatever reason, we are only able to measure a signal at the discrete time instants t=nTs for integer n∈Z, which we refer to as the discrete-time axis, we denote the sequence by sn=snTs, accordingly. Although the time axis is discrete in the variable n, in general sn can take real values.

In this context, we refer to the value xn0 as the sample of xt for t=n0Ts, n0∈Z. We refer to the spacing between samples in time Ts as the sampling period, and we define fs=1/Ts as the sampling frequency.

Finally, we often consider complex-valued signals and sequences such as

st=sRt+jsIt

sn=sRn+jsIn

where sRt and sIt are real-valued signals, sRn and sIn are real-valued sequences, and j=−1 is the standard notation for the unit imaginary number in the field of electrical engineering.

### Simple Signals

In this section, we define and visualize a number of simple signals.

#### Constant Signal

The simplest possible signal is the constant signal for which st=A for some constant value A∈R and all t∈R.

That is, the constant signal takes the same value for all time.

#### Rising Exponential Signal

Another simple signal is the rising exponential signal

rτt:=1−e−t/τt≥00t<0,  1

where τ>0 is called the time constant.

Since e−t/τ decays to 0 as t becomes large, i.e., limt→∞e−t/τ=0, we see that rτt approaches 1 as t becomes large, i.e., limt→∞rτt=1.

How fast rτt approaches 1 depends upon the time constant τ: larger values of τ cause rτt to approach 1 more slowly, and smaller values of τ cause rτt to approach 1 more rapidly.

This is illustrated in the simple numerical calculation and plot below.

import numpy as np
import matplotlib.pylab as plt

tau1 = 1
tau2 = 4
t=np.linspace(0,20,40)
r1 = 1-np.exp(-t/tau1)
r2 = 1-np.exp(-t/tau2)

label1 = "$\\tau="+str(tau1)+"$"
label2 = "$\\tau="+str(tau2)+"$"
plt.plot(t,r1,'s-',label=label1)
plt.plot(t,r2,'s-',label=label2)
plt.xlabel('$t$')
plt.ylabel('$r_{\\tau}(t)$')
plt.legend()
plt.show()

EXERCISE: Verify that, to achieve rτt≥0.99, we require t≥−ln0.01⋅τ≈4.6⋅τ.

Thus, a good rule of thumb is that a rising exponential signal essentially converges to its limit after five time constants. The duration 5τ is sometimes called the rise time of the rising exponential signal.

#### Unit Step Signal

Another simple signal is the unit step signal

ut:=1t>00t≤0.  2

That is, the unit step signal “turns on” after time t=0 after being “off” for t≤0.

Notice that the unit step can be viewed as a pointwise limit of the rising exponential signal as τ approaches 0, i.e., ut=limτ→0rτt for all t∈R.

On one hand, the mathematical expression for ut is simpler than that of the rising exponential signal, so ut can be a convenient approximation to the rising exponential signal for small τ.

On the other hand, ut is discontinuous at t=0, whereas the rising exponential signal is continuous for all t.

EXERCISE: Verify that

ut−τ=1t>τ0t≤τ.

In words, the delayed unit step signal ut−τ is off for t≤τ and on for t>τ.

#### Rectangular Signal

Another simple signal is the rectangular signal

rectt:=ut+12−ut−12.  3

EXERCISE: Verify that

rectt=1t<120t≥12.

In words, the rectangular signal rectt is off for t≤−1/2, turns on for −1/2<t<+1/2, and then turns off for t>+1/2.

EXERCISE: Determine a simple expression for rectt/T for T>0.

#### Dirac Delta Signal

Another signal that is actually defined in terms of a pointwise limit of functions is the Dirac delta signal

δt:=limϵ→0δϵt, t∈R,  4

where δϵt is a set of functions parameterized by ϵ>0 satisfying

−∞+∞δϵtdt=1, ϵ>0,  5

and

limϵ→0δϵt=+∞t=00t≠0.  6

The simplest such set of functions is given by

δϵt=1ϵrecttϵ=1ϵt<ϵ20t≥ϵ2.

That is, the δϵt functions are rectangular with width ϵ and height 1ϵ, so that they become narrower and taller as ϵ becomes smaller.

An important property satisfied by the Dirac delta signal is the sifting property, which states that for a signal st, t∈R, and for any τ∈R,

−∞+∞stδt−τdt=sτ.  7

In words, a Diract delta signal delayed by τ “sifts” out the value of the signal st for t=τ under the integral.

#### Sinusoidal Signals

Another very important class of signals are sinusoidal signals, such as cos2πf0t and sin2πf0t for all t∈R and a given f0>0. If we define T0=1/f0, we observe the following interesting property:

cos2πf0t+kT0=cos2πf0t+k2π=cos2πf0t, k∈Z.

In words, the sinusoidal signal cos2πf0t is periodic with fundamental period T0=1/f0.

The parameter f0 is called the frequency of the sinusoid, and represents the number of periods or cycles of the sinusoid in one second. The units of frequency are Hertz.

f_0 = 10
T_0 = 1/f_0
t = np.linspace(-2*T_0,2*T_0,4*20)
c = np.cos(2*np.pi*f_0*t)
s = np.sin(2*np.pi*f_0*t)

plt.plot(t,c,'s-',label='cos')
plt.plot(t,s,'s-',label='sin')
plt.xlabel('t')
plt.legend()
plt.show()

EXERCISE: Show that cos2πf0t+π/2=sin2πf0t, i.e., the cos “leads” the sin, or the sin “lags” the cos, by π/2 radians or 90∘.

#### Continuous-Time Complex Exponential Signals

The last class of simple signals we will mention are complex exponential signals of the form ej2πf0t for all t∈R and a given f0∈R, where j=−1.

Using Euler’s relation, we have

ej2πf0t=cos2πf0t+jsin2πf0t.  8

That is, the complex exponential signal has the cos and sin signals of a given frequency as its real and imaginary parts, respectively.

The following plot shows the cos and sin signals from the previous example in the complex plane (XY mode). The complex exponential cycles around the unit circle f0 times per second. Specifically, for this example with f0>0 and comparing to the previous figure, the complex exponential starts at the point 1,0 and rotates counter clockwise through the points 0,1, −1,0, and 0,−1. For f0<0, the complex exponential cycles clockwise around the unit circle −f0 times per second.

plt.plot(c,s,'s-')
plt.xlabel('Real Part')
plt.ylabel('Image Part')
plt.show()

EXERCISE: Verify using Euler’s relation that

cos2πf0t=12ej2πf0t+e−j2πf0t  9

and

sin2πf0t=12jej2πf0t−e−j2πf0t.  10

EXERCISE: Verify using Euler’s relation the following trigonometric identities

cos2πf1tcos2πf2t=12cos2πf1−f2t+cos2πf1+f2t  11

sin2πf1tsin2πf2t=12cos2πf1−f2t−cos2πf1+f2t  12

cos2πf1tsin2πf2t=12sin2πf1+f2t−sin2πf1−f2t  13

#### Discrete-Time Complex Exponential Sequences

We can think of a discrete-time complex exponential sequence as samples of a continuous-time complex exponential signal, but some subtleties arise.

Consider xt=ej2πft for a specific frequency f∈R. Let xn denote samples of this signal with sampling period Ts (sampling frequency fs), i.e., xn=xnTs.

Then

xn=ej2πftt=nTs=ej2πf/fsn

The ratio f/fs is called the normalized frequency of the discrete-time complex exponential, with units of cycles per sample.

If fs/f is an integer, the DT complex exponential is periodic, i.e., xn+kN0=xn for all n∈Z. In this case, the period N0=fs/f, the inverse of the normalized frequency.

Now consider ykt=ej2πf+kfst, for a specific frequency f∈R and a specific integer k∈Z. In words, the frequency of ykt is the frequency of xt plus an integer multiple k of the sampling frequency fs.

The corresponding discrete-time complex exponential is

ykn=yktt=nTs=ej2πf+kfs/fsn=ej2πkej2πf/fsn=ej2πf/fsn=xn

since ej2πk=1, k∈Z.

In words, the discrete-time samples corresponding to all continuous-time frequencies f+kfs are exactly the same. The continuous-time frequency f+kfs is said to alias to the frequency f, or the normalize frequency f/fs, for all k∈Z.

Another way to view this effect is that, for discrete-time sequences, the normalized frequency axis f/fs is periodic with fundamental period 1.

We therefore focus on discrete-time normalized frequencies on the interval −1/2,+1/2 as a convention, and we use the variable u to denote normalized frequencies.

## Signal Energy and Power

We will often deal with finite-energy or finite-power signals.

The energy of a signal st is defined as

Es=limT→∞−T/2+T/2st2dt  14

when the limit exists. The absolute value of the integrand is important for dealing with complex-valued signals.

The (average) power of a signal st is defined as

Ps=limT→∞1T−T/2+T/2st2dt  15

when the limit exists.

EXERCISE: Verify that the energy and average power of the rectangular signal rectt/T are E=T and P=0, respectively.

EXERCISE: Verify that the energy and average power of the complex exponential signal ej2πf0t are ∞ and 1, respectively.

Similarly, the energy and (average) power of a sequence sn are defined as

Es=limN→∞−N+Nsn2  16

and

Ps=limN→∞12N+1−N+Nsn2  17

when the limits exist, respectively.

## Correlation

For two signals xt and yt, we define the correlation as

<xt,yt>:=−∞+∞xty*tdt  18

For two sequences xn and yn, we define the correlation as

<xn,yn>:=−∞+∞xny*n  19

Two signals (sequences) are said to be orthogonal if there correlation is zero.

EXERCISE: Determine the correlation of rectt/T and rectt−τ/T, t∈R, for all values of τ,T∈R. For which values of τ, if any, are the two signals orthogonal?

EXERCISE: Determine the correlation of the signals ej2πf1t and ej2πf2t, t∈R, for all values of f1,f2∈R. For which values of f1,f2, if any, are the signals orthogonal?

EXERCISE: Determine the correlaton of the sequences ej2πu1n and ej2πu2n, n∈Z, for all values of u1,u2∈−1/2,1/2. For which values of u1,u2, if any, are the sequences orthongonal?

Note: Be careful with aliasing effects!

## Systems

A system models the processing of input signal(s) or sequence(s) into output signal(s) or sequence(s). For starters, we will explore systems with a single input and a single output. Systems that process continuous-time signals are called continuous-time systems, and systems that process discrete-time sequences are called discrete-time systems.

For simplicity at this point, we denote a system by H and its response to input xt by H{xt}, and we draw such systems as components in block diagrams and shown in Figure 2.

Figure 2: System block diagrams in continuous-time and discrete-time

### Simple Systems

#### Memoryless Operation

TBD

#### Delay / Advance

TBD

#### Time Limiter

TBD

### Linear Time-Invariant Systems

Linearity and time-invariance are two very useful system properties that are satisfied by many systems. Systems that satisfy both properties are called linear, time-invariant (LTI) systems.

#### Definitions

A system H is linear if it satisfies two properties:

- Scaling: If yt=H{xt}, then input H{axt}=ayt for all inputs xt and all constaints a∈C.

- Superposition: If y1t=H{x1t} and y2t=H{x2t}, then H{ax1t+bx2t}=ay1t+by2t, for all inputs x1t and x2t and all constants a,b∈C.

A system is time-invariant if, for yt=H{xt}, H{xt−τ}=yt−τ for all inputs xt and all constants τ∈R.

In other words, delaying an input produces the same delay in the corresponding output.

#### Convolution Defines the LTI Input-Output Relationship

The input-ouput relationship for an LTI system can be specified in general from ht=H{δt}, which is called the impulse response of the system.

Specifically we can determine yt=H{xt} through the convolution integral

yt=−∞+∞xτht−τdτ=:xt*ht  20

To see this, we rewrite the input as

xt=−∞+∞xτδt−τdτ

and apply the LTI properties of the system.

Note that he we are expressing the input as a superposition of many, simpler signals for which the input-output relationship is simple.

An equivalent relationship is

yt=−∞+∞xt−τhτdτ

#### Complex Exponentials are Eigenfunctions of LTI Systems

When we apply a complex exponential as the input to an LTI system, something very interesting and useful happens.

Mathematically, if ht is the impulse response of the LTI system H, applying the convolution intergral to the input ej2πf0t yields

−∞+∞ej2πf0t−τhτdτ=−∞+∞ej2πf0te−j2πf0τhτdτ=ej2πf0t−∞+∞hτe−2πf0τdτ

In words, if the input to an LTI system is any complex exponential with frequency f0, the output is a complex exponential with the exact same frequency.

For this reason, we call complex exponentials the eigenfunctions of LTI systems.

The effect of the LTI system on a complex exponential of frequency f0 is multiplication by the complex-valued scalar

Hf0:=−∞+∞hτe−j2πf0τdτ  21

Across all frequencies f∈R, which we refer to as the frequency axis, the system may have a different value of Hf, which we refer to as the frequency response.

EXERCISE: Argue that, if

xt=k=1Kakej2πfkt

is input to an LTI system with frequency response Hf, then the corresponding output is

yt=k=1KHfkakej2πfkt

The above exercise illustrates that convolution for LTI systems is particularly easy to work with if the input can be written as a sum of complex exponentials. This motivates us to represent as many signals this way as we can!

#### Discrete-Time LTI Systems

Very similar ideas apply to discrete-time systems as well.

The input-ouput relationship for an DT LTI system can be specified in general from hn=H{δn}, which is called the impulse response of the system.

Specifically we can determine yn=H{xn} through the convolution sum

yn=k=−∞+∞xkhn−k=:xn*hn  22

To see this, we rewrite the input as

xn=k=−∞+∞xkδn−k

and apply the LTI properties of the system.

Note that he we are expressing the input as a superposition of many, simpler signals for which the input-output relationship is simple.

An equivalent relationship is

yn=k=−∞+∞xn−khk

Mathematically, if hn is the impulse response of the DT LTI system H, applying the convolution intergral to the input ej2πu0n yields

k=−∞+∞ej2πu0n−khk=k=−∞+∞ej2πu0ne−j2πu0khk=ej2πu0nk=−∞+∞hke−j2πu0k

In words, if the input to an DT LTI system is any complex exponential with normalized frequency u0, the output is a complex exponential with the exact same frequency.

For this reason, we call DT complex exponentials the eigenfunctions of DT LTI systems.

The effect of the DT LTI system on a complex exponential of normalized frequency u0 is multiplication by the complex-valued scalar

Hej2πu0:=k=−∞+∞hke−j2πu0k  23

Across all frequencies u∈R, which we refer to as the discrete-time frequency axis, the system may have a different value of Hej2πu, which we refer to as the discrete-time frequency response.

EXERCISE: Argue that, if

xn=k=1Kakej2πukn

is input to an DT LTI system with frequency response Hej2πu, then the corresponding output is

yn=k=1KHej2πukakej2πukn

## Representation of Periodic Signals by Sums of Exponentials

This section focuses on periodic CT signals that satisfy xt+T0=xt, t∈R, for some T0>0 and periodic DT sequences that satisfy xn+n0=xn, n∈Z for some integer N0>0.

### Continuous-Time

If xt is a periodic signal with fundamental period T0>0, then we can represent it by the infinite sum of complex exponentials

xt=l.i.m.k→∞−kkakej2πkt/T0  24

and we may easily determine the coefficients ak, k∈Z via a correlation over any interval of length T0, for example,

ak=xt,1T0ej2πkt/T0=1T0−T0/2T0/2xte−j2πkt/T0dt  25

This representation is called the continuous-time Fourier series.

Remarks

- The Fourier series represents a periodic CT signal xt, t∈R by the DT sequence ak, k∈Z.

- The complex exponentials are harmonically related in the sense that the frequencies are integer multiples of f0=1/T0, which we call the fundamental frequency.

- For an LTI system with frequency response Hf, the output corresponding to the periodic input xt will be

l.i.m.k→∞−kkHkt/T0akej2πkt/T0

- that is, we apply the system to each component frequency, exploit the eigenfunction property of complex exponentials for LTI systems, and sum the results using linearity.

- The limiting behavior of the summation is in the sense of mean-square convergence, and is fairly technical. In particular, if we consider the finite summation

xKt=−KKakej2πkt/T0

- then xt=l.i.m.K→∞xKt means that

limK→∞−T0/2T0/2xt−xKt2dt=0

In words, the integral of the squared difference between the two signals approaches zero, rather than the difference betweeen the signals themselves going to zero for all t∈R.

TBD: Periodic square wave and Gibbs phenomenon, in Python plot

#### Discrete-Time

If xn is a periodic signal with integer fundamental period N0>0, then we can represent it by the finite sum of complex exponentials

xn=0N0−1akej2πk/N0n  26

and we may easily determine the coefficients ak, k=0,1,…,N0−1 via a correlation over any interval of N0 samples, for example,

ak=xn,1N0ej2πk/N0n=1N0n=0N0−1xne−j2πk/N0n  27

This representation is called the discrete-time Fourier series.

Remarks

- The Fourier series represents a periodic DT sequence xn by a finite set of coefficients ak,k=0,1,...,N0−1.

- The complex exponentials are harmonically related in the sense that their normalized frequences are k/N0, k=0,1,…,N0−1, where 1/N0 is the fundamental frequency.

- For an LTI system with frequency response Hej2πu, the output corresponding to the periodic input xn will be

k=0N0−1Hej2πk/N0akej2πk/N0n

- that is, we apply the system to each component frequency, exploit the eigenfunction property of complex exponentials for LTI systems, and sum the results using linearity.

- The Discrete Fourier Transform (DFT) of a sequence of length N is defined as

Xk=n=0Nxne−jπk/Nn  28

- for k=0,1,…,N−1. The discrete-time Fourier series coefficients are related to the DFT via the relationship Nak=Xk, so that the inverse DFT becomes

xn=1Nk=0NXkejπk/Nn  29

- This set of relationships are important because there is an efficient algorithm for computing the DFT (and inverse DFT) called the Fast Fourier Transform (FFT), particularly when N is a power of 2.

## Representation of Aperiodic Signals by Sums of Exponentials

Not all signals are periodic, but we still want to represent them as sums of complex exponentials to build upon the machinery above.

### Continuous-Time

The signal xt can be presented as an integral over a continuum of complex exponential signals over all frequencies via

xt=l.i.m.B→∞−B/2B/2Xfej2πftdf  30

where Xf denotes the continuous-time Fourier transform of xt, assuming the limit exists.

The CT Fourier transform can be defined in a fairly general sense as

Xf=l.i.m.T→∞−T/2T/2xte−j2πftdt  31

assuming the limit exists.

For compactness, we denote a CT Fourier transform pair by xt↔Xf.

Remarks

- The CT Fourier transform of a complex exponential ej2πf0t is stricly not well defined, unless we generalize to allow Dirac delta functions in the frequency domain. In that case, ej2πf0t↔δf−f0. We can obtain this intuition by time-limiting the complex exponential as ej2πf0trectt/T, computing the CT Fourier transform, and then taking the limit as T→∞.

- Energy and correlation can be computed in the frequency domain. Specifically,

Ex=−∞∞xt2dt=−∞∞Xf2df  32

<xt,yt>=−∞∞xty*tdt=∞∞XfY*fdf=<Xf,Yf>  33

- These results are called Parseval’s and Plancheral’s relationships, respectively.

- With this definition, convolution in the time domain becomes multiplication in the frequency domain. That is, if the Hf represents the frequency response of the LTI system H, i.e., the Fourier transform of the impulse response ht, then the input-output relationship yt=xt*ht is becomes Yf=HfXf.

- There are some extremely sophisticated mathematics with regard to integration and limit operations in order to provide a general notion of the Fourier transform and inverse Fourier transform. The commonly most general definitions relay on Lesbegue integration and mean-square limits, whereas most undergraduates are familiar with Riemann integration and pointwise limits.

### Discrete-Time

The sequence xn can be represented as an integral over a continuum of complex exponential sequences over all normalized frequencies in any interval of length 1, for example

xn=−1/21/2Xej2πuej2πundu  34

where Xej2πu denotes the discrete-time Fourier transform of xn as a function of the normalized frequency u.

The DT Fourier transform is defined as

Xej2πu=n=−∞∞xne−j2πun  35

assuming the limit exists.

For compactness, we denote a DT Fourier transform pair by xn↔Xej2πu.

Remarks

- Building upon our earlier discussion of discrete-time complex exponentials, we see that the DT Fourier Transform is periodic with period 1 on the normalized frequency axis. We tend to focus by convention on the interval u∈−1/2,1/2, but we should keep in mind this periodicity.

- The DT Fourier transform of a complex exponential ej2πu0n is stricly not well defined, unless we generalize to allow Dirac delta functions in the frequency domain. In that case, ej2πu0n↔δu−u0. We can obtain this intuition by time-limiting the complex exponential as ej2πunrectn/N, computing the DT Fourier transform, and then taking the limit as N→∞.

- Energy and correlation can be computed in the frequency domain. Specifically,

Ex=n=−∞∞xn2=−1/21/2Xej2πu2du  36

<xn,yn>=n=−∞∞xny*n=−1/21/2Xej2πuY*ej2πudu=<Xej2πu,Yej2πu>  37

These results are called Parseval’s and Plancheral’s relationships, respectively.

- With this definition, convolution in the time domain becomes multiplication in the frequency domain. That is, if the Hej2πu represents the frequency response of the DT LTI system H, i.e., the DT Fourier transform of the impulse response hn, then the input-output relationship yn=xn*hn becomes Yej2πu=Hej2πuXej2πu.