# Lecture 3 - Sampling & Reconstruction

EE-30023/31023, Department of Electrical Engineering, University of Notre Dame

## Summary

In this lecture, we will continue to refresh understanding on the background material from signals and systems. Specifically, we focus on sampling and reconstruction, multirate signal processing, and related practical aspects for analog-to-digital convertors (ADCs) and digital-to-analog converters (DACs).

## Motivation in Course Context

Based upon the modern radio architecuture introduced in the first lecture, we are now focused on the DACs and ADCs that provide us with the interface between the CT and DT worlds.

Figure 1: ADC architecture from https://www.analog.com/en/analog-dialogue/articles/the-right-adc-architecture.html

## Outline

- Analog-to-Digital Converter (ADC) Architecture

- Digital-to-Analog Converter (DAC) Architecture

- Sampling and Reconstruction of CT Signals

  - Frequency-Domain View of Sampling

  - Ideal Bandlimited Reconstruction and the Sampling Theorem

  - Aliasing, Nyquist Zones, and Anti-Alias Filtering

- Decimation and Interpolation of DT Sequences

  - Downsampling

  - Interpolation

  - DT Sampling Theorem

- Practical Interpolation Filters Require Oversampling

### Analog-to-Digital Conversion

Figure 2: Analog-to-Digital Converter (ADC) Architecture

At a high level, the ADC architecture in Figure 2 works well provided the following:

- First, the desired frequency components of the signal rt are sufficiently bandlimited, e.g., maximum frequency component less than fs/2M.

- Second, the filters ht and gn are practical approximations to low pass filters (LPFs) that pass the desired frequency content of rt and sn, respectively, and significantly attenuate other undesired frequency components. The wider the transition band of these filters, generally the more bandlimited we require rt to be.

- Third, the quantizer does not introduce too much distortion, e.g., y−qy<D for some small D for “most” values of the input y.

Note that quantization issues will be addressed in our next lecture.

### Digital-to-Analog Conversion

Figure 3: Digital-to-Analog Converter (DAC) Architecture

At a high level, the DAC architecture in Figure 3 works well provided the following:

- First, the quantizer does not introduce too much distortion, e.g., y−qy<D for some small D for “most” values of the input y.

- Second, the input signal yn is sufficiently bandlimited that its own periodic images, or the additional ones created through upsampling to produce xn, do not enter into the transition band or passband of the interpolation filters.

- Third, the filters gn and ht are practical approximations to low pass filters (LPFs) that pass the desired frequency content of yn and st, respectively, and significantly attenuate other undesired frequency components. The wider the transition band of these filters, generally the more bandlimited we require yn to be.

Note that quantization issues will be addressed in our next lecture.

## Sampling and Reconstruction of CT Signals

The time domain relationship for sampling is rather simple: given a CT signal st and sampling frequency fs>0, we define the sampling period as Ts=1/fs and set

sn=stt=nTs=snTs  1

The frequency domain relationship between the two samples that is more complicated.

### Sampling in the Frequency Domain

Let st↔Sf be a Fourer transform pair, and let sn=snTs, n∈Z for some sampling period Ts>0.

The frequency domain relationship between the CT signal and the DT sequence is as follows:

Sej2πu=1Tsk=−∞∞S(u−k)/Ts=fsk=−∞∞S(u−k)fs  2

This relationship demonstrates that sampling introduces three effects:

- Amplitude scaling by 1/Ts

- Periodic replication of the spectrum at integer multiples of the sampling frequency, i.e., replicas of Sf shifted by kfs∣k∈Z and added up

- Scaling of the frequecny axis u=f/fs

After we illustrate these effects via an example, we provide two alternative ways to develop the mathematical relationship.

#### Illustration

In the example below, st is a signal bandlimited to maximum frequency fm>0 and with Fourier transform Sf.

Figure 4: Illustration of the effects of sampling in the frequency domain.

We see from Figure 4 that there is no overlap between the periodic replicas Sf−kfs, k∈Z in Spf if

fs>2fm  3

#### Informal Proof

We define an intermediate CT signal spt=st⋅pt, where pt is an impulse train

pt=n=−∞∞δt−nTs

so that

spt=n=−∞∞snTspt−nTs

In the frequency domain, since pt is periodic with generalized Fourier transform

Pf=1Tsk=−∞∞δf−k/Ts

convolution in the frequency domain yields

Spf=1Tsk=−∞∞Sf−k/Ts

Finally, scaling the time access between CT and DT by 1/Ts via sn=snTs corresponds to scaling the frequency axis by Ts via

Sej2πu=Spu/Ts=1Tsk=−∞∞Su−k/Ts

#### Formal Proof

To establish the frequency-domain relationship, start with the CT inverse Fourier transform relationship

st=−∞∞Sfe−2πftdf

so that

sn=snTs=−∞∞Sfe−2πf/fsndf

The change of variables u=f/fs yields

sn=fs−∞∞Sufse−2πundu

which shows sn as a superposition of an infinite continuum of DT complex exponentials. However, these complex exponentials only live on the finite normalized frequency axis u∈−1/2,1/2 because e−2πu+kn=e−2πun for all k∈Z.

We therefore break up the infinite integral into a sum of integrals, one over each unit interval k−1/2,k+1/2, k∈Z. Specifically,

sn=fsk=−∞∞k−1/2k+1/2Sufse−2πundu

Finally, we define the changes of variables v=u−k and l=−k so that

sn=fsk=−∞∞−1/2+1/2Sv+kfse−2πv+kndv=−1/2+1/2fsl=−∞∞Sv−lfse−2πvndv

The last relationship is clearly the inverse DT Fourier transform relationship for sn, so that by inspection we have the desired result

Se2πvn=fsl=−∞∞Sv−lfs  4

### Bandlimited Interpolation and the Sampling Theorem

We say that a signal st is bandlimited to maximum frequency fm if Sf=0 for f>fm.

The example and informal proof above suggests that, despite periodic replication through the sampling process, we can recover a bandlimited CT signal from its samples provided there is no overlap of the replicas in the frequency domain, i.e., fs>2fm. This lower limit on the sampling frequency is called the Nyquist rate for the bandlimited signal.

#### Bandlimited Interpolation

Exact recovery simply consists of filtering the intermediate signal spt with a lowpass filter of the appropriate cutoff frequency and passband gain, and illustrated in the figure below.

Figure 5: Illustration of band limited interpolation in the frequency domain.

The lowpass filter passes and scales the replica centered at f=0 and eliminates the replicas centered at f=kfs, k∈Z−{0}.

Strictly speaking, the cutoff frequency for such a filter can be any frequency between fm and fs−fm; however, a convention is to set the cutoff frequency to be fs/2, as illustrated in Figure 5. With this convention, the ideal lowpass filter has frequency response

Hf=TsrectfTs  5

and impulse response

ht=sinct/Ts  6

The resulting reconstruction equation is called bandlimited interpolation, and takes the form

st=spt*ht=n=−∞∞snTsδt−nTs*sinct/Ts=n=−∞∞snTssinct−nTsTs  7

#### Sampling Theorem

Putting these observations together leads to the following result.

Theorem 1 (Sampling Theorem) If a continuous signal st is bandlimited to maximum frequency fm>0 and sampled with frequency fs>2fm, corresponding to the period Ts<1/2fm, then it can be reconstructed from its Ts-spaced samples as:

limN→∞n=−NNsnTs sinct−nTsTs=st

### Aliasing, Nyquist Zones, and Anti-Alias Filtering

If a CT signal st is not bandlimited, or even if the signal is bandlimited but the sampling frequency fs does not exceed its Nyquist rate, the periodic replicas Sf−kfs will overlap in the frequency domain. This effect is called aliasing.

Aliasing can be better understood if we define the Nyquist Zones, which consist of successive intervals of length fs/2 along the CT frequency axis. The Nyquist Zones are illustrated in the figure below.

Figure 6: Illustration of Nyquist zones.

Specifically, for positive k∈Z, the k-th Nyquist Zone is the interval k−1fs/2,kfs/2. For negative kZ, the k-th Nyquist Zone is the interval [kfs/2,k+1fs/2). All Nyquist Zones for k odd and positive alias into the k=1 Nyquist Zone, and all Nyquist Zones for k even and positive alias into the k=−1 Nyquist Zone. Conversely, all Nyquist Zones for k odd and negative alias into the k=−1 Nyquist Zone, and all Nyquist Zones for k even and negative alias into the k=1 Nyquist Zone.

To avoid aliasing, we often put a lowpass filter in front of the sampler, to limit the frequency content. Ideally, this would be a lowpass filter with cutoff frequency fs/2. Such a filter is called an anti-alias filter.

## Decimation and Interpolation of DT Sequences

In this section, we develop similar ideas for changing the sampling rate of DT sequences as for sampling of CT signals. The concepts are largely the same, although the notation is slightly different, and we have to keep in mind that the DT normalized frequency axis is periodic with period 1.

### Decimation

In some applications, we have fixed analog hardware that operates at a high sampling rate fs, and then we reduce the effective sampling rate in reprogrammable digital hardware or software. Decimation or Downsampling by a factor M∈Z+ has input-output relationship

yn=xnM  8

which looks very similar to the relationship for sampling a CT signal to produce a DT sequence. The downsampler keeps every M-th sample and discards those inbetween.

Figure 7 shows a block diagram for the downsampling operation and illustrates a good way to think about its operation relative to CT sampling.

Figure 7: Block diagram for downsampling relative to CT sampling.

In particular, if we imagine a CT signal xt for which the sequence xn represents the samples xnTs, downsampling yields the samples yn=xnMTs, i.e., samples spaced with larger period MTs or smaller sampling frequency fs/M.

Using the previous frequency-domain relationships for sampling from CT to DT, we can, after some algebraic manipulations, obtain the following DT relationship for downsampling

Yej2πu=1Mm=0M−1Xej2πu−m/M  9

Again, we have replication of the spectrum, except there are only M replicas within a period of length 1 in the DT frequency axis.

As in the CT case, aliasing can occur, we can define DT Nyquist zones, and we can apply anti-alias or decimation filters before downsampling to reduce the effects of aliasing.

### Interpolation

We often want to recover a DT sequence from its downsampled version, or to increase the effective sampling rate in reprogrammable hardware or software, partiularly in order to produce samples for fixed analog hardware that operates at a high sampling rate. This interpolation is accomplised in two steps, upsampling followed by filtering.

#### Upsampling

Upsampling by a factor M∈Z+ has input-output relationship

wn=xn/Mn=0,±M,±2M,…0otherwise  10

The upsampler inserts M−1 zero samples between input samples. Another way to write down this relationship mathematically is

wn=k=−∞∞xkM δn−kM

which is analogous to CT pulse train conversation.

Figure 8 shows a block diagram for the upsampling operation and illustrates a good way to think about its operation relative to CT pulse train conversion.

Figure 8: Block diagram for upsampling relative to CT pulse train conversion.

In particular, we imagine a DT sequence signal xn that we plan to convert back into continuous time with sample spacing MTs. Intead of getting there direclty with an impulse train with spacing MTs, we can achieve the same result by first upsampling and then converting to an impulse train with spacing Ts. Equating the two resulting spectra Wpf, we can derive the input-output relationship for upsampling to be

Wej2πu=Xej2πuM  11

Since upsampling scales the time axis by the factor M, it scales the frequency axis by the factor 1/M. This is yet another instance in which we have to be conscious about the periodic nature of the DT frequency axis and the DT Fourier transform. This is illustrated in Figure 9 for M=2.

Figure 9: Illustration of periodicity in the frequency domain and upsampling.

Imagining that xn represents samples of a CT signal xt, this example illustrates that upsampling xn by 2 is not the same as sampling xt at twice the frequency. There are extra frequency components in Xej2πu2, for example, those that are centered at ±1/2, that are called images.

#### Interpolation Filtering

To eliminate the images introduced by upsampling, we again apply a lowpass filter, however, this time in discrete time. For interpolation by a factor of M, the convention is to apply an ideal DT lowpass filter with cutoff frequency 1/2M. The frequency response of this filter is

Hej2πu=MrectuM  12

for u∈−1/2,1/2 and periodic with period 1, and its impulse response is

hn=sincn/M.  13

### DT Sampling Theorem

A DT sequence xn is bandlimited to normalized frequency um>0 if Xej2πu=0 for um<u<1/2. Note that this definition is slightly different that in CT, again because the DT frequency axis is periodic with period 1.

Theorem 2 (Reconstruction) If a DT sequence xn is bandlimited to maximum frequency 0<um1/2 and downsampled by the integer factor M such that M<1/2um, then it can be reconstructed through bandlimited interpolation of its M-spaced samples as

limK→∞k=−NNxkM sincn−kMM=xn

### General Sample Rate Conversion

In many cases, we want to convert to a fractional sampling rate. If the sample rate conversion that we want to achieve is a factor M/N, the usual approach is to first interpolate by the factor M and then decimate by the factor N.

## Practical Interpolation Filters Require Oversampling

The final, and practically most important, topic for this lecture addresses practical implementation of interpolation filters. The ideal lowpass filters discussed so far are non-causal and infinite duration, making them impossible to implement in practice.

In this section we consider more general filters for the CT interpolatio

st=l.i.m.N→∞n=−NNsnTs ht−nTs.  14

Two classic examples are the zero-order hold, for which ht=rectt/Ts, and linear interpolation, which is developed in the problem set.

Another possibility is to time-limit the sinc pulse by setting

ht=rectt/2LTs sinct/Ts,

which maintains 2L sampling periods about t=0. This multiplication in the time domain corresponds to convolution in the frequency domain. The smaller we seek to make the integer parameter L, and thereby reduce the filtering delay of the interpolation, the wider the dispersion of the ideal lowpass response in the frequency domain.

For any one of these non-ideal filters, let f1 denote the largest frequency in the passband, and let f2 denote the smallest frequency in the stopband. The band of frequencies f1,f2 is often call the transition band of the lowpass filter.

Returning to our first illustration for a signal st bandlimited to maximum frequency fm, to ensure that the spectrum replica Sf is passed by the filter, we require f1>fs. However, to ensure that the replica Sf−fs is not passed by the filter, we require f2<fs−fm. If fm is fixed, and we want to widen the transition band by increasing f2, then the only way to ensure good reconstruction, i.e., st≈st, is to increase the sampling rate fs as we increase f2. This often translates into fs being considerably larger than the Nyquist rate 2fm, and is in general called oversampling.

## Quantizer Parameters and Input-Output Relationships

A quantizer is a function that takes a continuous-valued input and products a discrete-valued output. It is generally a non-invertible function.

Figure 10 illustrates a fairly general quantizer with N∈Z+ quantization levels. Often N is chosen to be a power of 2.

Figure 10: Illustration of a generic quantizer.

### General Parameterization

Mathematically, we define a set of N+1 values b0=−∞<b1<b2<…<bN−1<bN=∞ along the input axis, and we refer to them as the quantization region boundaries. These values partition the input space into the intervals

Rn=(bn−1,bn]

for n=1,2,…,N, which are called the quantization regions.

We also define the set of N values a1<a2<…aN along the output axis, and refer to them as the quantizer representation points.

The functional definition of the quantizer is then qy=an if y∈Rn, n=1,2,…,N.

The minimum representation point a1 and the maximum representation point aN define the output range aN−a1 of the quantizer.

### Minimum Mean-Square Error Quantization

If we are given a probability distribution on the input fYy, then we can consider choosing the quantization region boundaries and representation points to minimze EqY−Y2. This is called minimum mean-square quantization.

Although a solution to this problem does not exist in general, there is an iterative approach that will converge to at least a local minimum that is called the Lloyd-Max Algorithm. The iterative algorithm alternates between improving the quantization region boundaries and the representation points. Specifically:

- Given the quantization representation points {a1,a2,…,aN}, we select the quantization region boundaries according to correspond to the midpoints between representation points, i.e.,

bn=a1+an2

for n=1,2,…,N−1.

- Given the quantization region boundaries {b0,b1,…,bN}, we select the quantization representation points to be the conditional mean of the random variable over the quantization region, i.e.,

an=EY|Y∈Rn

for n=1,2,…,N.

### Uniform Quantization

For many practical implementations, the quantization regions R2,R3,…,RN−1 are set to be the same length Δ>0, so that the quantization region boundaries satisfy

bn=bk−1+Δ  15

for k=2,3,…,N1.

Furthermore, the representation points of the internal quantization regions are the midpoints of these intervals. Specifically,

an=bn+bn−12=bn−1+Δ2n=2,…,N−2bn−Δ2n=1bn+Δ2n=N−1  16

The output range of such a quantizer is then aN−a1=N−1Δ.

We can define a standard, K-bit uniform quantizer with output range 1 as the function

qKy=⌊ry+12+1/2r⌋/r−12−12<y<+12+12y≥+12−12y≤−12  17

where r=2K−1. For convenience, the quantizer representation points can be easily represented the integers 0,1,…,2K−1 or their binary representations.

This quantizer limits the input to ±1/2. If we want to limit the input and output to ±A, then we can scale the quantizer by applying 2AqKy/2A.

The input-output relationships for this standardized and scaled standardized uniform quantizer as illustrated in the Python code below.

import numpy as np
import matplotlib.pyplot as plt

def q(y,K):
    z = np.zeros(np.size(y))
    for i in range(np.size(z)):
        if y[i] >= 0.5:
            z[i] = 0.5
        elif y[i] <= -0.5:
            z[i] = -0.5
        else:
            r = np.power(2.0,K)-1.0
            z[i] = np.floor( r*(y[i]+0.5 + 1.0/r/2.0 ) ) / r - 0.5
    return z

K = 4                   # 4 bits, 16 levels
M = 100*np.power(2.0,K) # 100 samples per quantization region
A = 1                # Expand range of quantizer to \pm A

y_values = np.arange(-2*A,+2*A,A/M)
qKy = 2.0*A*q(y_values/(2.0*A),K)

plt.plot(y_values,qKy,'-')
plt.plot(y_values,y_values,'-.')
plt.xlabel("$y$")
plt.ylabel("$2 A q_K(y/(2A))$")
plt.show()

## Effect of Uniform Quantization on Sinusoidal Signals

u0 = 1/4
n = np.arange(0,128.0/u0,1)
y = np.cos(2*np.pi*u0*n)

K=8
A=2.5
yq=2.0*A*q(y/(2.0*A),K)

nfft=512
u=np.fft.fftshift(np.fft.fftfreq(nfft))
Y=np.fft.fftshift(np.fft.fft(y,nfft))
Yq=np.fft.fftshift(np.fft.fft(yq,nfft))
plt.plot(u,10.0*np.log10(np.abs(Y)**2),'-')
plt.plot(u,10.0*np.log10(np.abs(Yq)**2),'--')
plt.xlabel("$u$")
plt.ylabel("$Y(e^{j2\pi u})$ (dB)")
plt.axis([-1/2.0, 1/2.0, -80.0, 60.0])
plt.show()

## Take Aways

- Quantization converts a continuous-amplitude input into a discrete-valued output, and it inherently introduces some distortion.

- It is important to consider the output range of the quantizer relative to our input signal amplitude to ensure that we obtain a low error representation

  - If the input signal amplitude is too small, quantization noise will dominate the output.

  - If the input signal amplitude is too large, clipping distortion will dominate the output.