# Lecture 09 - Amplitude Modulation

EE-30023/31023, Department of Electrical Engineering, University of Notre Dame

## Summary

In this lecture, we will develop carrier modulation and demodulation theory to be able to translate baseband (low-frequency) waveforms into passband (high-frequency) waveforms and back.

## Motivation in Course Context

In the context of radio communication system block diagram below, we now have the understanding of how to generate and process low-frequency signals with our DAC and ADC, respectively. As we will see, in a few lectures, to emit wireless signals via an antenna, we will need to translate the low-frequency signals into high-frequency signals to which the antenna is tuned.

Figure 1: TBD.

The key take away from the antennas module is that, depending upon the geometry, they are “resonant” to particular frequencies. Unless the antennas are impractically large, they tend to prefer high-freqeuency, or passband signals.

Thus, to continue building our radios we need to develop techniques and associated RF circuits to translate from baseband signals to passband signals and back.

## Outline

- Baseband and Passband Signals

- Carrier Modulation

- Carrier Demodulation

## Signal Classes

To begin, let us be clear about the two classes of signals upon which we focus.

### Baseband Signals

A baseband signal is either strictly bandlimited about f=0, or has most of its signal energy concentrated about f=0.

Specifically, let xt be a finite-energy signal with Fourier transform Xf, and for a given frequency B>0, let

βB=f≤B​Xf2dfEx

Here βB is the fraction of the signal energy that lies in the frequency range f∈−B,+B. Often, we want to identify the frequency B that has 90%, 95%, 99%, (and so on) of the signal energy, which we would call the 90%−, 95%−, 99%− (and so on) bandwidth, respectively. If xt has a 100% bandwidth B, then it is strictly bandlimited to max frequency B.

### Passband Signals

By contrast, a passband signal is one that has most of its energy centered about another frequency fc≫0, which is often called the center frequency of the signal.

Specifically, let

βfc,B=f−fc≤B​Xf2dfEx

and

βpB=maxfcβfc,B

The notions of 9x%-passband bandwidth about a center frequency fc are analogous to those of $9x%-baseband bandwidth about DC.

### Bandwidth

By convention, bandwidth of a real-valued signal is a measure of the range of positive frequencies of a signal that contain most of its energy. So a baseband signal that has most of its energy in the interval f∈−B,+B is said to have (baseband) bandwidth B, while a passband signal that has most of its energy in the intervals f∈−fc−B,−fc+B and f∈fc−B,fc+B is said to have passband bandwidth 2B.

## Amplitude Modulation

We begin with a real-valued, baseband signal mt, t∈R. Let B>0 denote the baseband bandwidth of this signal, so that its Fourier transform Mf, f∈R, satisfies Mf=0 for f>B.

Suppose that we wish to represent mt as a passband signal that is centered at frequency fc>0.

### Idea #1

Looking at the problem strictly in the frequency domain, a fairly intuitive idea to pursue is to translate the signal as follows

S1f=Mf−fc

The corresponding relationship in the time domain is

s1t=mtej2πfct

that is, multiplication by a complex exponential with frequency fc. Although simple mathematically, the result is a complex-valued signal that we cannot implement in real circuits.

### Idea #2

Now suppose that we simply force the signal to be real by taking the real part of s1t, i.e.,

where the last step is valid because we assumed that mt is real-valued.

The diagram below is often used as a block diagram representation of the modulation.

Figure 2: TBD

This multiplication is often called a modulation, and the signal ct is often called the carrier signal.

In the frequency domain, the relationship is

S2f=12Mf−fc+Mf+fc

which can be obtained either my considering the convolution of Mf with the (generalized) Fourier transform of the cosine 12δf−fc+δf+fc or by considering the Fourier transform of s2t=12s1t+s1*t.

In words, the modulation creates two copies or images of the spectrum of mt, namely Mf−fc centered at +fc and and Mf+fc centered at −fc.

We note that the highest frequency of the modulated signal st is fc+B.

To ensure that the images do not overlap in the frequency domain, we require fc>2B, but very often fc≫2B.

Some common terminology associated with this modulation scheme includes:

- mt is called the message signal or modulating signal

- ct is called the carrier signal

- fc is called the carrier frequency or center frequency

- mt modulates the amplitude of the carrier signal, so the method is called amplitude modulation. More specifically, it is called double sideband suppressed carrier (DSB-SC) modulation

## Amplitude Demodulation

Now suppose we have the opposed problem, named, we want to convert from a passband signal s2t back to a baseband signal mt.

### Idea #1

Looking at the problem strictly in the frequency domain, a fairly intuitive idea to pursue is to translate the signal as follows

M1f=S2f+fc

The corresponding relationship in the time domain is

m1t=s2te−j2πfct

In this case, the signal is complex-valued, and there is extra frequency content centered at f=−2fc, so that m1t≠mt.

## Idea #2

To remove the extra frequency content centered at f=−2fc, we can simply lowpass filter the result of the previous idea.

Figure 3: TBD

## Idea #3

Finally, we consider m3t=Rem2t, which yields

Figure 4: TBD

Another way to view the overall system is illustrated in the figure below.

Figure 5: TBD

The combination of modulation and the first stage of demodulation is equivalent to modulation of mt by

cos22πfct=121+cos2π2fct

via trigonemtric identities.

This leads to three images of Mf centered at f=0 and f=±fc, the latter two of which are filtered out by the low-pass filter.

## Discrete-Time Implementation

To emulate the modulation in discrete time, either for digital circuit implementations or software implementations on a computer, we consider a sampled version of st with sampling period Ts>0.

To satisfy the sampling theorem for st, we require that the sampling frequency fs=1/Ts satisfy fs>2fc+B, which implies that we also satisfy the sampling theorem for mt and ct.

If we define the discrete-time sample sequences as sn=snTs, mn=mnTs, and cn=cnTs, respectively, for n∈Z, the modulation becomes

sn=mn⋅cn=mncos2πfcTsn=mncos2πfc/fsn

where uc=fc/fs is the normalized frequency of the DT carrier signal.

Let’s continue with some simple example code to generate a discrete-time, carrier-modulated waveform.

import numpy as np
import matplotlib.pylab as plt

B = 20000.
fc = 800000.
fs = 5*2.*(fc+B)
Ts = 1/fs
n = np.arange(2*fs/B,dtype=int)
m = np.cos(2*np.pi*(B/fs)*n)
c = np.cos(2*np.pi*(fc/fs)*n)
s = m*c

plt.subplot(311)
plt.plot(n,m,'-')
plt.ylabel("$m[n]$")
plt.subplot(312)
plt.plot(n,c,'-')
plt.ylabel("$c[n]$")
plt.subplot(313)
plt.plot(n,s,'-')
plt.ylabel("$s[n]$")
plt.xlabel("Sample Index")
plt.show()

K=8192
S = np.square(np.abs(np.fft.fft(s,K)))
plt.plot(np.arange(0,fs,fs/K),S,'-')
plt.ylabel("$|S(f)|^2$")
plt.xlabel("Frequency (Hz)")
plt.axis([fc-3*B,fc+3*B,0.,1.1*np.max(S)])
plt.show()

We note that this implementation model only considers the highest frequency signal of the modulator. To implement the demodulator as well, we would require fs>22fc+B.