# Lecture 11 - Electromagnetic Waves and Antennas

EE-30023/31023, Department of Electrical Engineering, University of Notre Dame

## Summary

In this lecture, we will recall the solutions to Maxwell’s equations in the far field to understand the salient aspects of propagation of electromagnetic (EM) waves from a transmit antenna to a receive antenna. The summary is distilled from the book Constantine A. Balanis, Antenna Theory - Analysis and Design, 4th Edition, John Wiley & Sons, 2016. Available online via ND Libraries’ subscription to Knovel.

## Motivation in Course Context

Now that we have prototyped a working wireline communication link, we seek to cut the wire and go wireless. We have looked at digital communication networks and wired links from the top down, and now we will begin to look at radio links from the bottom up.

As illustrated in the figure below, we will begin with developing developing circuit-level models for how a current or voltage input to a transmit antenna translates into a current or voltage output from a receive antenna.

Figure 1: Current transfer function model of a transmit antenna, electromagnetic wave propagation, and receive antenna.

As we will see, the physics of electromagnetic (EM) waves and a number of antenna parameters affect this transfer function.

## Lecture Outline

- Electromagnetic Wave Propagation

- Antenna Parameters

- RF Link Budget

## Electromagnetic Wave Propagation

Section Outline

- Maxwell’s Equations

- Far-Field Solution

- Example: Infinitesimal Dipole

### Maxwell’s Equations

Electromagnetic (EM) waves satisfy Maxwell’s equations. These equations relate the electric field E and the magnetic field B, both of which are three-dimensional vector functions that can vary with both space r=x,y,z and time t,

Er,t=Exr,t,Eyr,t,Ezr,t

Br,t=Bxr,t,Byr,t,Bzr,t

An EM wave in free space influenced by a scalar charge density function ρr,t and a vector current density function Jr,t satisfies the partial differential equations

or the wave equations

where ϵ0 is the permittivity of free space, μ0 is the permeability of free space,, and

c0=1μ0ϵ0=2.99792458×108 m/s

is the speed of light in free space.

Rectangular and spherical coordinates, the differential operator ∇, divergence (dot product with ∇), curl (cross product with ∇), and Laplacian (multiplication by ∇2=∇⋅∇) are summarized in the appendix.

### Far-Field Solutions for a Transmitting Antenna

An antenna is an electrical transducer that converts a time-varying current into an EM wave, or vice versa.

If we locate an antenna at the origin of our coordinate system, and apply a time-varying input current or voltage to the antenna, the geometric shape of the antenna affects a particular time-varying charge density ρ and current density J about the origin. The charge and current densities become “sources” in Maxwell’s equations, and we refer to the antenna as a transmit antenna because these sources generate the resulting EM waves that propagate away from the origin.

Solving Maxwell’s equations for the resulting electric field Er,t and magnetic field Br,t becomes quite involved, even for the simplest transmit antenna geometries. However, we can make some general, albeit approximate, observations about the fields and radiation patterns around the antenna if we consider the distance from the origin r=∥r∥ to be sufficiently large. This regime is called the far field of the antenna.

#### Time-Harmonic Wave

For simplicity, we consider a sinusoidal input with a single frequency f Hertz, and we write the resulting electric and magnetic field intensities in the forms

Er,t=ReE0rej2πft

Hr,t=ReH0rej2πft

where E0r and H0r represent complex-valued phasor vectors. Notice that this representation allows us to separate the spatial and temporal / spectral dependence of the fields.

#### Electric Field

In spherical coordinates, for large r, solutions to Maxwell’s equations for the electric field phasor vector are approximately of the form

E0r,θ,ϕ≃0,Eθθ,ϕ,Eϕθ,ϕe−jkrr

where k=2π/λ is the wave number. Notice that the electric field has no radial component under this approximation, i.e., E0⋅r=0. Furthermore, notice that the dependence upon r and the dependence upon θ,ϕ are separated.

Substituting this form of the electric field phasor vector, we obtain, for example, the following form of the elevation component of the time-varying EM wave

Eθr,θ,ϕ,t=1rEθθ,ϕcos2πft−kr+∠Eθθ,ϕ

We recognize this as a decaying sinusoidal wave propagating away from the origin in the direction of r with speed c0, frequency f, and wavelength λ=c0/f.

The Python code below animates one electric field component for a frequency of f=50 MHz over a radial range of r=5λ to r=10λ over one period t∈0,1/f. We see that the wave propagates away from the origin, i.e., in the direction of increasing r.

import numpy as np
import matplotlib.pyplot as plt
from IPython import display
import time

c=2.99792458e8
f=50e6
lamb=c/f
r=np.linspace(5*lamb,10*lamb,200)
t=np.linspace(0,1.0/f,50)
for n in range(0,np.size(t)):
    E=np.cos(2*np.pi*f*t[n]-2*np.pi*r/lamb)/r
    plt.plot(r,E,'-')
    plt.axis([5*lamb,10*lamb,-1/lamb/5,1/lamb/5])
    plt.xlabel('Distance $r$')
    plt.ylabel('Electric Field $E_\\theta$')
    display.clear_output(wait=True)
    display.display(plt.gcf())
    time.sleep(0.001)
    plt.clf()

<Figure size 480x384 with 0 Axes>

#### Polarization

If we also consider the orthogonal component of the electric field

Eϕr,θ,ϕ,t=1rEϕθ,ϕcos2πft−kr+∠Eϕθ,ϕ

several interesting cases arise:

- Linear Polarization - Only one component, or two orthongonal components with phases that are integer multiples of π, i.e.,

∠Eϕθ,ϕ−∠Eθθ,ϕ=nπ, n=0,1,2,…

- Circular Polarization - Two orthogonal components with the same magnitude, i.e.,

Eθθ,ϕ=Eϕθ,ϕ

and phase differences that are odd multiples of π/2, i.e.,

∠Eϕθ,ϕ−∠Eθθ,ϕ=+12+2n)π, n=0,1,2,…clockwise−12+2n)π, n=0,1,2,…counterclockwise

- Elliptical Polarization - There are two cases in which elliptical polarization arises. First, the phases can satisfy either of those for circular polarization, but the field component magnitudes are not the same. Second, the phase difference is not equal to any multple of π/2, regardless of the magnitudes.

Some nice visualizations of these difference polarizations are available at: http://hyperphysics.phy-astr.gsu.edu/hbase/phyopt/polclas.html

#### Magnetic Field

Similarly, for large r, solutions to Maxwell’s equations for the magnetic field phasor vector are approximately of the form

H0r,θ,ϕ≃1η00,−Eϕθ,ϕ,Eθθ,ϕe−jkrr

where η0=μ0/ϵ0 is the intrinsic impedance of the free-space medium. Notice that H0 also has no radial component, i.e., H0⋅r=0, and that the electric field and magnetic field are orthogonal, E0⋅H0=0, under this approximation.

These observations are consistent with the “right hand rule” that says the cross product of the electric and magnetic fields points in the direction of propagation, i.e., E0×H0∝r.

#### Power Density

Finally, the average power density vector of the EM wave is

i.e., all of the power density points in the radial direction of propagation r. The average power density vector is obtained by averaging the instantaneous Poynting vector Wr,t=Er,t×Hr,t over one period T=1/f of the EM wave.

The average power radiated by the antenna can be written as the surface integral of the average power density W0r,θ,ϕ over a sphere S of radius r. Specifically,

Note that at the point r on the sphere S, r/∥r∥ represents the unit normal vector of the sphere at r, and the differential area is dA=r2sinθdθdϕ.

### Example: Infinitesimal Dipole Antenna

In free space, consider an infinitesimal dipole antenna of length l with current I0 uniformly distributed over the interval z∈−l/2,l/2. We now summarize the results of solving Maxwell’s equations for this antenna from Balanis, Chapter 4.

#### Electric Field

The electric field in spherical coordiates at the point r=r,θ,ϕ (stricly away from the source itself) satisfies

For kr≫1, which is referred to as the far field region, this result can be approximated as

E0r,θ,ϕ≃0,jη0kI0lsinθ4π,0e−jkrr

The propgating wave then takes the form

Eθr,θ,ϕ,t=1rkI0lsinθ4πcos2πft−kr+π2

which shows the dependence on the magnitude I0 and frequency f of the input current. In particular, if the input current oscillates at frequency f, then the output EM wave oscillators at the same frequency f.

Since all the other field components are zero, we see that the electric field oscillators only along the θ dimension, perpendicular to the direction of propagation. Such an electromagnetic wave is said to be vertically polarized.

#### Magnetic Field

Similary, the magnetic field intensity satisfies

For kr≫1, this result can be approximated as

H0r,θ,ϕ≃0,0,jkI0lsinθ4πe−jkrr

As summarized earlier, we see that Hϕ=1η0Eθ in this far-field regime.

#### Power Density

The average power density vector for the dipole satisfies for kr≫1

W0r,θ,ϕ≃η08I0lλ2sin2θr2,0,0

Therefore, the power radiated by the antenna is

Recalling that a current IOcos2πft through a resistor of R Ω dissipates power P=I02R/2, we see that we can model the radiated EM power of the antenna as that of a resistance

Rr=η02π3lλ2

which is called the radiation resistance of the antenna. This observation allows us to model the antenna as a circuit element, which is a concept we will revisit later in this lecture.

## Antenna Parameters

Section Outline

- Radiation Intensity

- Beamwidth

- Directivity

- Antenna Efficiency

- Gain, Realized Gain

- Effective Length and Area

### Radiation Intensity

The radiation intensity for an antenna in the far field is related to the electric field via

where we have used the result

E0r,θ,ϕ≃0,Eθθ,ϕ,Eϕθ,ϕe−jkrr

The average power radiated by the antenna can be computed from the radiation intensity via

Prad=02π0πUθ,ϕsinθdθdϕ

#### Example: Isotropic Source

An isotropic source is an idealized radiator with constant radiation intensity Uθ,ϕ=U0 for all 0≤θ≤π and 0≤ϕ≤2π. The average power radiated by such a source is

Prad=02π0πU0sinθdθdϕ=4πU0

or

U0=Prad4π

#### Units: Watts per Steradian

The units of radiation intensity are Watts per steradian (sr), or Watts per unit solid angle.

One steradian is defined as the solid angle with its vertex at the center of a sphere of radius r that is subtended by a spherical surface area equal to r2, i.e., the same area as that of a square of side length r. Since the surface area of a sphere is 4πr2, there are a total of 4π steradians 4πr2/r2.

Figure 2: Illustration of steradians.

With these units, the average power can be computed as

Prad=Ω​UdΩ

which is an integral over all 4π steradians with dΩ=sinθdθdϕ.

### Beamwidth

A beamwidth of an antenna is generally defined as the angular separation between two identical points on opposite sides of the pattern maximum. For example, two important beamwidths include:

- The Half-Power Beamwidth (HPBW) is the angle between two directions in which the radiation intensity is one-half the value of its maximum.

- The First-Null Beamwidth (FNBW) is the angular separation between the first nulls of the radiation intensity.

#### Example

Supposed the normalized radiation intensity of an antenna is represented by

Uθ=cos2θcos23θ

import numpy as np
import matplotlib.pyplot as plt

theta=np.linspace(0,np.pi,100)
U=np.cos(theta)**2 * np.cos(3*theta)**2
plt.plot(theta/np.pi,U,'-')
plt.plot(theta/np.pi,0.5*np.ones(np.size(theta)),'--')
plt.plot(theta/np.pi,np.zeros(np.size(theta)),'--')
plt.xlabel('Elevation $\\theta/\\pi$')
plt.ylabel('Radiation Intensity $U(\\theta)$');

from scipy.optimize import fsolve

func = lambda ang : 0.5 - np.cos(ang)**2 * np.cos(3*ang)**2
theta_hp=fsolve(func, 0.05)[0]
print(theta_hp)

0.2508478997797948

Based upon the above root-finding method, the HPBW of the radiation intensity is approximately θ≃0.25 radians or θ/π≃0.08. This is verified below in the plot of Uθ on a decibel (dB) scale.

plt.plot(theta/np.pi,10*np.log10(U),'-')
plt.plot(theta/np.pi,10*np.log10(0.5)*np.ones(np.size(theta)),'--')
plt.xlabel('Elevation $\\theta/\\pi$')
plt.ylabel('Radiation Intensity $U(\\theta)$ (dB)');

### Directivity

The directivity of an antenna is the ratio of the radiation intensity in a given direction from the antenna to the radiation intensity averaged over all directions, or equivalently, the radiation intensity in a given direction from the antenna relative to that of an isotropic radiator. Mathematically,

Dθ,ϕ:=Uθ,ϕU0=4πUθ,ϕPrad

If the direction is not specified, the direction of maximum radation intensity is implied, so that

Dmax=maxθ,ϕDθ,ϕ

### Efficiency and Gain

When we consider the interface between electronic circuits and an antenna, a variety of losses can arise.

Figure 3: Input and output antenna terminals.

Figure 4: Reflection, conduction, and dielectric losses in an antenna.

The total efficiency of an antenna e0 attempts to capture all of these losses. It is a product of the following efficiency factors:

- er - the reflection (mismatch) efficiency caused by potential mismatch between a transmission line impedance and the antenna impedance

- ecd - the antenna radiation effiency, which is often difficult to compute but can be measured experimentally

With these parameters, we can relate the radiated output power to the input power of the circuits, and define appropriate notions of gain.

The gain captures the transfer of power from the input of the antenna, without the transmission line, to the EM wave, and can be expressed as

Gθ,ϕ=ecdDθ,ϕ

The realized gain captures the transfer of power from the input of the antenna, with the transmission line, to the EM wave, and can be expressed as

Greθ,ϕ=erecdDθ,ϕ

When the transmission line characteristic impedance (often Z=50 Ω) and the antenna input impedance are matched, then er=1 and the two gains are the same.

### Impedance Models for Antennas

Since we will be integrating antennas with electric circuits, we can model them as circuit elements.

#### Transmit Antenna

Figure 5: Transmit antenna model.

Figure 6: Transmit antenna circuit model.

Here, the quantities represent the following:

- Zg=Rg+jXg is the source impedance

- Za=Ra+jXa is antenna impedance, where Ra=Rr+RL

- Rr is the radiation resistance of the antenna

- RL is the loss resistance of the antenna

Note that maximum power is delivered to the antenna when we have conjugate matching, i.e.,

Ra=Rg  Xa=−Xg

In this case, the power delivered to the radiation resistance is

Pr=Vg28RrRr+RL2

#### Receive Antenna

Figure 7: Receive antenna model.

Figure 8: Receive antenna circuit model.

Here, the quantities represent the following:

- ZT=RT+jXT is the load impedance

- Za=Ra+jXa is antenna impedance, where again Ra=Rr+RL

- Rr is the radiation resistance of the antenna

- RL is the loss resistance of the antenna

Note that maximum power is delivered to the load when we have conjugate matching, i.e.,

Ra=RT  Xa=−XT

In this case, the power delivered to the load resistance is

PT=VT28RT

We note that the impedance is a function of frequency, so that we can only match over a small bandwidth.

### Vector Effective Length

The vector effective length (or vector effective height) of an antenna is represented by

ler,θ,ϕ=0,lθθ,ϕ,lϕθ,ϕ

and provides a convenient way to convert between scalar currents and voltages and the vector electric field quantities.

Specifically, when viewed as a transmitting antenna with input current amplitude Iin, the electric field of an antenna with vector effective length ler,θ,ϕ can be expressed as

E0r,θ,ϕ=−jη0kIin4πler,θ,ϕe−jkrr

When viewed in receiving mode, the effective length determines the open circuit voltage amplitude at the terminals of the antenna given an electric field impinging on the antenna:

Voc=E0r,θ,ϕ⋅ler,θ,ϕ

We stress the the input current to the transmitting antenna and the voltage output from the receiving antenna are oscillating at frequency f, but these relationships focus on the current amplitude Iin and voltage amplitude Voc and the electric field phasor vector E0.

### Effective Area or Aperture

The effective area or aperture of a receive antenna is written in equation form as

Ae=PTWi=IT2RT/2Wi

where PT is the power delivered to the load, Wi is the power density of the incident EM wave, and RT is the load resistance. The effective aperture represents an area that, when multiplied by the radiation power density of the incident wave, gives the power received by the load resistance.

Balanis goes into more detail on effective area, and one of the key results is that the maximum effective area Ae,max of an antenna is related to its maximum directivity Dmax by

Ae,max=λ24πDmax

### Example: Half-Wavelength Dipole

A dipole with length l=λ/2 oriented along the z axis has the following parameters.

- Electric field in the far-zone:

Er≃0

Eθ≃jη0I02πcosπ2cosθsinθe−jkrr

Eϕ=0

- Radiation intensity:

Uθ=η0I028π2cosπ2cosθsinθ2≃η0I028π2sin3θ

Figure 9: Three-dimensional antenna pattern for a half-wavelength dipole antenna.

- Total radiated power:

- Maximum directivity: Since Uθ is maximized for θ=π/2, we have

Dmax=4πUπ/2Prad≃42.435≃1.643

- Maximum effective area:

Ae,max=λ24πDmax≃λ24π1.643≃0.13λ2

- Radiation resistance and impedance:

Rr=2PradI02≃η04π2.435≃73

Zin=73+j42.5

To reduce the imaginary part of the impedance to zero, the antenna is matched or reduced in length until the reactance vanishes. The latter is most commonly used in practice for half-wavelength dipoles. Depending upon the radius of the wire, the length of the dipole for the first resonance is about l=0.47λ (for thicker wire) to 0.48λ (for thinner wire).

Reactance of a half-wavelength dipole antenna

## RF Link Budget

Section Outline

- Friis Transmission Equation

- WiFi Example

- Polarization Loss

- Vertical-Circular Example

### Friis Transmission Equation

The Friss transmission equation relates the power received to the power transmitted between two antennas that are separated by a distance R>2D2/λ, where D is the largest dimension of either antenna. In other words, we focus on situations in which each antenna is in the far field of the other. The scenario is illustrated and parameterized in the figure below.

Figure 10: Friis model.

As usual, we consider a sinusoidal input signal with power Pt and frequency f applied to the terminals of the transmit antenna, so that the generated EM wave propagates with frequency f and has wavelength λ=c/f.

#### Polarization Matched Antennas

For a non-isotropic transmitting antenna, the power density in the direction θt,ϕt can be written as

Wt=PtGtθt,ϕt4πR2=etPtDtθt,ϕt4πR2

where Gtθt,ϕt is the gain, Dtθt,ϕt is the directivity, and et is the efficiency of the antenna.

The effective area of the receive antenna is

Ar=erDrθr,ϕrλ24π

where Drθr,ϕr is the directivity and er is the efficiency of the receive antenna. Assuming the antennas are polarization matched, the receive power is then

The ratio of the received power to the transmit power is

The last term is often referred to as the (free-space) propagation loss. Note that this loss increases as the square of the distance R and / or the square of the frequency f.

If we can point the antennas so that they present maximum gain toward each other, then the best case ratio becomes

PrPtmax=Gt,maxGr,maxc04πfR2

#### Example

WiFi devices supporting IEEE Standard 802.11 can operate in the 5 GHz Industrial, Scientific, and Measurement (ISM) band.

- What is the free-space loss at a propagation distance of 50 m?

L=λ4πR2=c4πfR2≈3×108 m/s4π5×109 Hz50 m2≈9.119×10−9=−80.4 dB

- If the recommended receive power level of WiFi is −65 dBm, and the nominal transmit power is 15 dBm, what is the required product of antenna gains, assuming the antennas are polarization matched?

GtGrdB>−65 dBm−15 dBm−−80.4 dB=0.4 dB

- Since WiFi can also operate in the 2.4 GHz ISM band, would the received power be larger or smaller than at 5 GHz for the same antenna gains and distance?

So there is about 6.4 dB less loss at 2.4 GHz that at 5.0 GHz.

#### Polarization Loss

We observe from the above relationships that if the antennas are not pointing at each other in the direction of maximum directivity or gain, then the output received power will be less than the maximum possible.

In fact, even if we point the antennas directly at each other with maximum directivity, there can be a loss due to polarization mismatch, which we call polarization loss.

In the extreme case, if the transmit antenna is vertically polarized, and the receive antenna is horizonally polarized, the receive electric field will generate no current in the receive antenna, and the receive power is zero.

Recall that we denoted the electric field as a complex-valued phasor vector. If we let Etθt,ϕte−jkrr and Erθr,ϕre−jkrr denote the electric field phasors for the transmit and receive patterns in the far field, respectively, the polarization loss factor is the normalized magnitude-square of the dot product, i.e.,

Etθt,ϕt⋅Erθr,ϕr∥Etθt,ϕt∥∥Erθr,ϕr∥2

In the case of the transmit antenna being vertically polarized and the receive antena being horizontally polarized, we have Etθt,ϕt⋅Erθr,ϕr=0.

More generally, the polarization loss is smallest if the two field phasors are co-linear, in which case the loss factor is 1, or 0 dB. For this case, we call the antennas polarization matched.

#### Example

Suppose a transmit antenna is vertically polarized, so that Eθ,tθt,ϕt=1 normalized and Eϕ,tθt,ϕt=0.

If the receive antenna is circularly polarized in the clockwise direction, we have

Eθ,rθr,ϕr=Eϕ,rθr,ϕr=1/2

normalized, and

∠Eϕ,rθr,ϕr−∠Eθ,rθr,ϕr=+π/2

for example.

The polarization loss factor is then

1,0⋅12,ejπ/222=12

or 3 dB.

## Appendix: Vector Calculus Definitions

Consider the vector r=x,y,z=r,θ,ϕ illustrated in the figure below.

Figure 11: Three-dimensional coordinate system in rectangular coordinates x,y,z or spherical coordinates r,θ,ϕ.

The angle 0≤θ≤π is called the elevation angle, and the angle 0≤ϕ≤2π is called the azimuth angle.

### Coordinate Transformations

The transformation from rectangular to spherical coordinates is

rθϕ=sinθcosϕsinθsinϕcosθcosθcosϕcosθsinϕ−sinθ−sinϕcosϕ0xyz

and the inverse orthogonal coordinate transformation from spherical to rectangular is

xyz=sinθcosϕcosθcosϕ−sinϕsinθsinϕcosθsinϕcosϕcosθ−sinθ0rθϕ

Note that the transformation matrices are transposes of each other, i.e., they are orthogonal matrices.

### Differential Operations in Rectangular Coordinates

The del vector differential operator is defined as

∇=∂∂x,∂∂y,∂∂z

so that the divergence is

∇⋅A=∂Ax∂x+∂Ay∂y+∂Az∂z

the curl is

∇×A=1,0,00,1,00,0,1∂∂x∂∂y∂∂zAxAyAz=∂Az∂y−∂Ay∂z,∂Ax∂z−∂Az∂x,∂Ay∂x−∂Az∂y

and the Laplacian is

∇2=∇⋅∇=∂2∂x2+∂2∂y2+∂2∂z2

### Differential Operations in Spherical Coordinates

The del vector differential operator is defined as

∇=∂∂r,1r∂∂θ,1rsinθ∂∂ϕ

so that the divergence is

∇⋅A=1r2∂∂rr2Ar+1rsinθ∂∂θAθsinθ+1rsinθ∂Aϕ∂ϕ

the curl is

∇×A=1rsinθ∂∂θAϕsinθ−∂Aθ∂ϕ,1r1sinθ∂Ar∂ϕ−∂∂rrAϕ,1r∂∂rrAθ−∂Ar∂θ

and the Laplacian is

∇2=1r2∂∂rr2∂∂r+1r2sinθ∂∂θsinθ∂∂θ+1r2sin2θ∂2∂ϕ2