# Lecture 14 - PAM / QAM Demodulation

EE-30023/31023, Department of Electrical Engineering, University of Notre Dame

Thus far we have discussed modulating signals up to passband and then demodulating them back down to baseband. In a perfect world, this process would be simple. In practice, however, distortion of the signal occurs due to numerous factors like the underlying circuitry, the propagation environment, etc. In this lecture we will begin to develop a method of correctly determining the transmitted signal despite this distortion.

## Minimum Distance Detection in Signal Space

Before thinking about any specific distortion in a signal, we can use a very general signal space model to understand conceptually how we will detect the correct signal.

Suppose that our transmitter emits one of M waveforms s0t,s1t,...,sM−1t and that our receiver obtains a distorted signal rt. We know that rt is a distorted version of one of the smt possible transmitted signals, so we must develop a method to figure out which one the received signal is. It turns out that the natural decision criterion (often the mathematically optimal one) is to select the signal which is closest in signal space to the received signal. We call this method of detection Minimum Distance Detection.

Our decision m indicates which of the smt signals corresponds to rt using minimum distance. We calculate m by first computing the energy difference between each possible transmitted waveform and the received waveform, and then selecting the waveform with the minimum energy difference as the correct one. Mathematically, it looks as follows:

m=argminm=0,1,2,...,M−1∞∞rt−smt2dt

which looks a lot like an inner product. Therefore, we can rewrite it in two ways:

m=argminm=0,1,2,...,M−1rt−smt2

using the norm or

m=argminm=0,1,2,...,M−1⟨rt−smt,rt−smt⟩

using inner product notation. We’ll opt to continue with the latter.

Because we know inner products have the property of linearity, we can separate this into several separate inner products as follows:

m=argminm=0,1,2,...,M−1⟨rt,rt−smt⟩−⟨smt,rt−smt⟩=argminm=0,1,2,...,M−1⟨rt,rt⟩−2Re{⟨rt,smt⟩}+⟨smt,smt⟩

Notice that the inner product between rt and itself is independent of m, so it does not need to factor into the minimization calculation. Often the inner product between smt and itself can be precomputed because the set of waveforms is known at the receiver. So, in order to compute the minimum distance we only need to calculate the middle term, the inner product of rt and smt.

Now we can look at how to do this computation in signal space. Suppose {s1,s2,...,sM} exist in a signal space spanned by a smaller set of orthonormal waveforms {φ1,φ2,...,φK} where K<M. Then

rt=k=1Krkφkt

r=r1r2...r3T

and

smt=k=1Msmkφkt

r=sm1sm2...smKT

We see that for each continuous time signal we have a finite dimensional vector in our signal space that corresponds to it. We can then leverage Parseval’s Relation (which says that the energy between the difference of two signals squared is the same as the norm squared of their vector representation) to find the energy difference between both signals.

Therefore,

m=argminm=0,1,2,...,M−1rt−smt2

is equivalent to

m=argminm=0,1,2,...,M−1r−sm2

Now we’ve shown that if we have a signal space representation of a waveform, then minimum distance detection of signals is the same as minimum distance detection of symbols (i.e., euclidean distance!) The signal space view is much easier to work with than the signal view because we no longer need to use integrals.

A minimum distance detector using correlators.

We see that we take the received waveform and separately multiply it by each of the basis waveforms and then integrate (i..e, perform the inner product) to get the vector coefficients and thus a signal space representation of the waveform. We can then perform minimum distance detection on this vector with the set of transmit waveform vectors to select the correct one.

This is one way of implementing a minimum distance detection receiver. The multiplication followed by integration is called correlation, which is why many texts will call the above design a correlation receiver.

Note

Correlation is just a special case of a more general notion that we’ve already developed: the inner product.

### Matched Filter Receiver

There is another way to design the correlator that we just saw by replacing the correlation with a convolution. We know that the inner product ⟨rt,φkt⟩=−∞∞rtφk*tdt can be implemented as follows:

A matched filter sampling at time t=0.

We can verify that this is true mathematically. Let qt=φk*−t.

In general, the convolution yt for some signal rt and qt is yt=−∞∞rτqt−τdτ and therefore y0=−∞∞rτq−τdτ. We can substitute our qt to get

y0=−∞∞rτφk*τdτ

Clearly, this shows that the correlation can be implemented by performing a convolution and then sampling at time t=0.

We can then design our receiver as follows using matched filters instead of correlations:

A minimum distance detector using matched filters.

Causality

If φkt is causal, then the time-reversed version φk*−t is anti-causal. Relationship between φkt and φk*−t.. This filter is not realizable, but we can implement this filter if we delay by some T to make it causal. This is possible because if filter is finite in time. We can then just account for this delay in the system. If φkt≠0 for t∈0,T0 then we can implement matched filter as: Matched filter with time adjustment..