# Lecture 18 - Passband and Baseband Equivalents

EE-30023/31023, Department of Electrical Engineering, University of Notre Dame

There are a few details about amplitude modulation that we did not cover in previous lectures. Many of these details will help us think in both passband and baseband. We have been thinking of signals in terms of their actual representation in circuits (i.e., thinking about I and Q channels), but today we will focus on their mathematical representation.

## Baseband to Passband

Let xbt↔Xbf be a potentially complex-valued baseband waveform with max freq W Hz We can write xbt=Re{xbt}+Im{xbt} to represent the signal’s real and imaginary parts.

The passband equivalent signal with center frequency fp and phase φp is

xpt=2Rexbtej2πfp+φp

xp is another signal that corresponds to xb based on two parameters: some center frequency (fp) and phase (φp). This is only a mathematical operation, so we are not thinking about a mixer or demodulator or any hardware - just multiplication by a complex exponential. You’ll recall that the Re{⋅} operator introduces the complex conjugate as follows:

xpt=12xbtej2πfpt+φp+xb*te−j2πfpt+φp

We can also think in fourier domain, recalling that xb*t↔Xb*−f, so the passband fourier transform is

Xpf=12ejφpXbf−fp+e−jφpXb*−f+fp

A view of the copies generated by modulating up to passband.

Because we don’t want the aliases of the signal to overlap with each other, we require that fp>W.

Note

−∞∞xbt2dt=−∞∞xpt2dt We see that normalization by 2 preserves the energy of the signal. Not all textbooks will do this normalization - some may only use the real part of the signal - which leads to the energy between passband and baseband signals to be different.

It is essential to understand that these signals contain the same information. We can go from xb to xp and back without losing any information.

## Passband to Baseband

Since xpt is real valued, its Fourier transform is hermitian symmetric, i.e,

Xpf=Xp*−f

Xpf even about f=0

∢Xpf odd about f=0

This means that the signal is completely specified by its positive frequencies. If we need Xp−f, we can find it by looking at Xpf. Therefore we can say

Xp+f=Xpfuf

where

uf=1f≥012f=00f<0

If we shift Xp+ down to baseband, we recover Xbf with some phase and scaling factor. More specifically,

Xbf=2e−jφpXp+f+fp

In the above equation, we undo the scaling factor of 2, phase offset, and frequency offset which were all introduced as we went to passband. Xp+ is called the analytic signal corresponding to Xpf.

Aside

The typical notation for the positive frequency content of a signal M is M+f=Mfuf. Correspondingly, the negative frequency content is specified by M−f=Mfu−f. Therefore, we have Mf=M+f+M−f If mt is real-valued, M+f=M−*−f so either M+f or M−f completely specifies Mf. We use the positive frequency content for the analytic signal by convention, but in principle the negative frequency content would work as well.

We can think of uf as the frequency response of some real filter. Consider the LTI frequency response

Hf=jsgnf=e−jπ/2f>00f=0ejπ/2f<0

The time domain representation of this filter is

ht=1πt

Since ut is not physically realizable, we need to create some real filter that has a similar response.

### Hilbert Transform

The Hilbert Transform is the output of the system ht with the signal as the input

The Hilbert Transform

We write the Hilbert transform as follows:

Xhf=XfHf=−jsgnfXf

xht=−∞∞xtπt−τdτ

We use this filter because we can do a much better job approximating it than we can ut. The analytic signal can then be written in terms of the Hilbert transform as

X+f=12Xf+jXhf

When f is positive, Xhf is −jXf. −j×j=1, and we get that X+f=Xf. When f is negative, Xhf is jXf. j×j=−1, and we get that X+f=0. Clearly, then, we see that X+f only contains the positive frequency content of X.

We can similarly write

X−f=12Xf−jXhf

to represent the negative frequency content of X.

We now have a better way to recover our signal from passband. Recall that we said

Xbf=2e−jφpXp+f+fp

We can write Xp+ in terms of the Hilbert Transform to get

Xbf=12e−jφpXff+fp+jXhf+fp

The same can be shown in the time domain

xbt=12e−jφpxpte−j2πfpt+jxphte−j2πfpt=12xpt+jxphte−j2πfpt+φp

We have now shown an alternative way of coming from passband down to baseband.

Recovering baseband signal using the Hilbert Transform

The other way we have done so is with some mixer to shift the whole signal down and an LPF on the baseband signal

Recovering baseband signal using the Hilbert Transform

We now know that these two systems are equivalent with one simple caveat: in the Hilbert case, the filtering happens at passband. In the other case, the filtering happens at baseband. There are pros and cons to both depending on the specific application.

We will see applications of this concept next time when we study single-sideband modulation.