# Lecture 21 - Thermal Noise

EE-30023/31023, Department of Electrical Engineering, University of Notre Dame

In our radio circuits, particularly in the receive circuitry, we will need to account for the fact that components with resistive elements exhibit thermal noise. The high-level concept is illustrated in the figure below.

Practical resistor model.

An ideal resistor when shorted exhibits zero voltage drop. However, a real resistor when shorted exhibits a random voltage drop associated with vibrations of the molecules in the resistor. We treat the voltage source wt as a random process to deal with uncertainty.

Real resistances arises in the antennas, transmission lines, transistor gates, and other circuit components in the receiver, and we need to develop models for the noise and circuit and algorithm designs to reduce its impact on link performance.

## Random Process Model

Some signals are modeled as random processes because they exhibit significant uncertainty. In particular, if we were to short a real resistor and measure the voltage across it for two distinct periods of time, even under the most controlled settings the two voltage signals would be unequal almost everywhere! For an ideal resistor, the voltages would be identically zero for both periods.

### Time and Ensemble Domains

To address uncertainty, we characterize statistical behavior of the random signal averaged over extremely long periods of time or over many, many measurements. Fortunately, for many random signals of interest, these two characterizations are consistent with each other.

More specifically, we utilize two variables to define a random signal, a time variable t∈R and a measurement variable ω∈Ω.

- The set Ω captures all possible outcomes of the measurements, in this case all possible measured waveforms, each as a function of time t∈R. In the general theory of probility and random processes, Ω is called the sample space or ensemble.

- For each fixed ω0∈Ω, the measurement wt,ω0 is a conventional signal waveform.

- For each fixed t0∈R, the measurement wt0,ω is a random variable on the sample space Ω.

- For simplicity of notation, we often drop the dependence upon ω when it is clear from the context that the signal is modeled as a random process and would not otherwise cause confusion.

### Time, Ensemble, and Sample Averages

Consider a random process xt,ω, t∈R, ω∈Ω and function gx, x∈R that represents a quantity we may want to measure or compute related to the process.

For a given measurement ω0∈Ω, we can compute the time-average value of gxt,ω0 as

gxt,ω0:=limT→∞1T−T/2+T/2gxt,ω0dt

Since g(xt,ω0 is a function of the sample space, it is in general a random variable.

On the other hand, for a given time t0∈R, we can compute the ensemble average value of gxt0,ω as

Egxt0,ω:=−∞∞gxft0xdx

where ft0x represents the probability density function of the random variable xt0,ω, ω∈Ω. Since Eg(xt0,ω can be a function of time, it is in general a waveform. The ensemble average is also called the expectation or expected value in various contexts.

For the important class of stationary and ergodic processes, it turns out that gxt,ω0 and Egxt0,ω are the same, the former taking the value with probabilty one, i.e., almost all ω∈Ω, and the latter taking the value for all values of time t0∈R.

Finally, either of the above quantities can be estimated or computed via sample averages. In particular, if we obtain a given measurement ω0∈Ω at times t1,t2,…,tN∈R, N∈Z+, then we expect

1Nn=1Ngxtn,ω0→g(xt,ω0

as N→∞. Similarly, for a given t0∈R, if we obtain multiple measurements ω1,ω2,…,ωN∈Ω, N∈Z+, then we expect

1Nn=1Ngxt0,ωn→Egxt0,ω

as N→∞.

Subtle details about the notions of convergence for sequences of random variables and random processes are left for more advanced courses.

## Thermal Noise in the Time Domain

With the random process concepts and notation in place, we can now summarize some important properties of thermal noise.

### Average Value

To begin, thermal noise has the property that time and ensemble average values of the signal are zero. Specifically,

wt,ω)=0

with probability one, i.e., almost all ω∈Ω, and

Ewt,ω=0, t∈R.

### Correlation

Next, if we compute the long-term correlation of the thermal noise with a delayed version of itself, we obtain zero, except for extraordinarily small delays. Specifically, we can define the time average correlation function

wt+τ,ωw*t,ω:=limT→∞1T−T/2T/2wt+τ,ωw*t,ωdt

and find that it is zero for τ>ε and almost all ω∈Ω, i.e., with probability one. Note that we have allowed for complex-valued noise sources for generality.

Similarly, we can define the ensemble average correlation function

Rwt,τ:=Ewt+τ,ωw*t,ω=−∞∞−∞∞w1w2*ft+τ,tw1,w2dw1dw2

and find that it is zero for $|| > $, where ft+τ,tw1,w2 denotes the joint probability density function for the random variables wt+τ,ω and wt,ω.

For the important class of stationary and ergodic processes, we have

wt+τ,ωw*t,ω=Rwt,τ=Rw0,τ

with probability one, and this function only depends upon τ not t.

As we will explain later ε is of the order of picoseconds, which is extremely challenging to measure anyway. So we can approximately say that the correlation function is zero except for τ=0, which we address next.

### Average Power

Finally, the average power of the noise signal Pw is non-zero, and corresponds to the correlatons for τ=0, either the time average wt,ω2 for almost all ω∈Ω, i.e., with probability one, or the ensemble average Ewt,ω2 for all t∈R.

## Random Processes in the Frequency Domain

### Spectrum Analyzer

A spectrum analyzer is a device used to measure average signal power as a function of frequency. The operation of a spectrum analyzer can be modeled as shown in the figure below, with the idea being that ϵ is small and T is large.

Spectrum analyzer operation.

The first block is a complex-valued bandpass filter centered at frequecny f0 with squared magnitude response

Hf0,ϵf2=1ϵf−f0<ϵ20otherwise

We note that the impulse response of such a filter is complex-valued because the frequency response is not Hermitian symmetric.

### Power Spectral Density

It is convenient to define the power spectral density (PSD) as the limit

Swf:=limϵ→0limT→∞Swf,ϵ,T

If wt is a deterministic signal with Fourier transform Wf, then Swf0,ϵ,T is just a real number, and

limϵ→0limT→∞Swf0,ϵ,T=Wf02

If wt is a random process, then Swf0,ϵ,T is a random variable for each f0. For many relevant random process models that satisfy the properties of being stationary and ergodic, Swf is a well-defined, real-number for f∈R.

Several comments are in order:

- The total average power in the signal is the integral of the PSD

Pw=−∞∞Swfdf

Thus, the PSD represents the power density in the infinitesmal interval of length df centered at f.

- If yt is a filtered version of wt, i.e,. yt=gt*wt for some determinstic impulse response gt↔Gf, then

Syf=Gf2Swf

Thus, only the magnitude response and not the phase response of the filter Gf affects the PSD.

- In practice, we measure Swf,ϵ,T as a noisy approximation to Swf for ϵ small and T large.

- The power spectral density is the (generalized) Fourier transform of the autocorrelation function, i.e.,

Swf=−∞+∞Rw0,τ e−j2πfτdτ

## Thermal Noise in the Frequency Domain

Experiments (Johnson, 1928) and theory (Nyquist, 1928) suggest that the thermal noise in a real resistor can be modeled as a zero-mean Gaussian random process with PSD satisfying

Swf=4hfRehf/kT−1

where:

- R Ohms is the resistance

- h=6.6×10−34 J⋅s is Planck’s constant

- k=1.38×10−23 J/K is Boltzman’s constant

- T is temperature in Kelvin

Now, if we expand the denominator in a Taylor series about f=0, we have

ehf/kT−1=hkTf+12hkT2f2+⋯

The first term dominates if f≪kT/h. In this case,

Swf≃4kTR

i.e., the noise PSD is roughly flat as a function of frequency. With such an approximation, the autocorrelation function is approximately Rw0,τ=4kTRδτ.

At room temperature (T≃300 K), the upper limit for this approximation is about 6 THz. This bandwidth suggests that the autocorrelation function Rw0,τ decays to zero for τ on the other of picoseconds, as mentioned earlier.

As we will see later, the power dissipated by this noise source in a matched load will be kT per Hz. At room temperature (T≃300 K), this quantity is often expressed as

10log10kT×1000 dBm/Hz=10log101.38×10−23 J/K×300 K×1000≃−174 dBm/Hz

## Bandlimited Thermal Noise

Suppose that we filter thermal noise with an ideal bandpass filter as shown in the figure below, with B≪kT/h so that the input noise PSD is flat over the frequencies of interest.

Bandpass filter.

The total average power in the output noise wBt will be

PwB=−B/2B/2Swf df=4kTRB

i.e., the total average noise power is proportional to the bandwidth.

By applying the sampling theorem, we can represent wBt by its samples wn=wnTs with sampling period Ts=1/2B, i.e.,

wBt=limN→∞n=−N+Nwn sinct−nTsTs

Since wBt, t∈R is a random process, the sequence wn, n∈Z is a sequence of random variables. It turns out that these random variables are zero-mean, independent Gaussian random variables with variance PwB. Thus, we have

wn∼N0,4kTRB

This allows us to model noise processes in the computer, as we will address in a followup lecture.

## Signal-to-Noise Ratio

Now suppose that we have a voltage source st with a source resistance Rs driving a load resistance Rl. We assume for simplicity that any reactances have been matched out. Both the source and load resistances are in thermal equilibrium and exhibit thermal noise sources wst and wlt, which would be independent and uncorrelated noise sources.

Thermal noise in a modeled in a circuit.

If the input signal voltage is bandlimited with power

Ps=−B/2+B/2Ssfdf

the source signal st dissipates power

PsRlRs+Rl2Rl−1=PsRlRs+Rl2≤Ps4 Rs

in the load resistance Rl. The inequality is achieved for Rl=Rs.

In the same bandwidth, the source resistance thermal noise source wst dissipates power

4kTRsBRlRs+Rl2Rl−1=4kTBRs⋅RlRs+Rl2≤kTB

in the load resistance Rl, and similarly, the load resistance thermal noise source wlt dissipates power

4kTRlBRsRs+Rl2Rs−1=4kTBRs⋅RlRs+Rl2≤kTB

Thus, the two resistors are in thermal equilibrium and dissipate the same amount of power in each other. We do not consider that the source thermal noise dissipates power in the source resistor, or that the load thermal noise dissipates power in the load resistor. The inequalities in each case are achieved by Rl=Rs.

The signal-to-noise ratio (SNR) in the load resistance over the frequency band is then

SNR:=Ps4kTRsB=−B+BSsfdf−B+BSwsfdf

## References

- Henry W. Ott, Noise Reduction Techniques in Electronic Systems, 2nd Edition, John Wiley & Sons, 1988.

- Behzad Razavi, RF Microelectronics, 2nd Edition, Prentice Hall, 2011.