# Lecture 22 - Noise Figure

EE-30023/31023, Department of Electrical Engineering, University of Notre Dame

In this lecture, we will continue our discussion of noise in electronic circuits. Building upon the observations about thermal noise in the previous lecture, we will consider how noise accumulates along a cascade of circuits, and what design criteria we can apply in order to manage the noise end-to-end.

## Motivation in Course Context

We continue to focus on the receiver circuits because the desired received signal can be relatively weak due to propagation effects, and the thermal noise in the receiver can become significant relative to the received signal.

There are multiple sources of noise in the circuit.

As the figure above suggests, there are many sources of thermal noise in the receiver circuits. We need to come up with tools for modeling how the noise accumulates, and how we prevent it from overwhelming the received signal through updates to the receiver design.

## Power Gain / Loss

Each device of the radio receiver will be modeled as a gain or loss in power from the input to the output. Specifically, if the input power is Pi and the output power is Po, then the power transfer relationship will be defined as

G=PoPi

If G<1, the device will exhibit a power loss, while if G>1, the device will exhibit a power gain. On a dB scale, gains will be positive values, and losses with be negative values.

For filters, we will primarily focus on the loss in the passband. Terms such as insertion loss are commonly associated with passive filters, and we naturally want to keep the insertion loss as small as possible.

For mixers, we will primarily focus on the gain / loss of the translation from the input band to the output band. Terms such as conversion gain and conversion loss are commonly associated with mixers.

## Noise Figure

Suppose a device with gain / loss G has at its input the output of a signal generator with signal power Ps and a noise power Ns. The input signal-to-noise ratio (SNR) is then

SNRi=PsNs

The gain or loss of the component affects the input signal and noise in the same way. However, the component may exhibit internal thermal noise that contributes to the output signal.

Assuming the internal thermal noise has power Nd and is independent of the input signal and the input noise, the output noise power will be

No=G⋅Ns+Nd

so that the output SNR will be

SNRo=G PG Ns+Nd

The noise figure of the component is defined as the ratio of the input SNR to the output SNR, i.e.,

NF:=SNRiSNRo=1GNoNs

Solving for No, we have

No=NF⋅G⋅Ns=G⋅Ns+Nd

so that

Nd=NF−1 G Ns

The scenario and parameterization of the noise figure is illustrated in the figure below.

Example of noise figure relationship.

We stress a few items at this point:

- The relationships above relate the power of the internal noise to the power of the signal generator noise; they are not saying that the signals themselves are related, and in fact, they are modeled as indepedent random processes.

- The gain and noise figure are defined relative to a specific signal generator noise, often thermal or Johnson noise for which Ns=kTB where k is Boltzman’s constant, T is the temperature in Kelvin, and B is the (baseband) bandwidth in Hz.

- When we consider multiple devices in the following sections, all of their noise figures must be defined relative to a common signal generator noise input, and their internal noise powers are computed relative to the same input noise source.

## Cascaded Components

### Two Components

Suppose we cascade two components with gains / losses G1 and G2 and noise figures NF1 and NF2, respectively, as shown in the figure below. What is the gain / loss G12 as well as the noise figure NF12 of the cascade?

Noise figure with two devices cascaded.

First of all, we clearly have

G12=G1⋅G2

To determine the cascade noise figure, let N1 and N2 denote noise powers at the respective device outputs, and let

Nd1=NF1−1G1NsNd2=NF2−1G2Ns

be the source-input-referred internal noises of the two devices. We stress the importance of referencing these internal noises to the signal generator input noise, the idea being that the noise figure of each device individually is defined as if the device itself was connected ot the signal generator intead of being cascaded.

From above, we have

N1=NF1⋅G1⋅Ns

and

N2=G2⋅N1+Nd2=G2⋅N1+NF2−1G2⋅Ns

The first term represents the component of the output noise due to the source input noise, and the second term represents the component of the output noise due to the internal noise of the second device.

Substituting the equation for N1 into the equation for N2, we have

N2=NF1+NF2−1G1G1G2Ni=NF12⋅G12⋅Ni

Thus, the noise figure of the cascade satisfies

NF12=NF1+NF2−1G1

### Multiple Components

More generally, suppose we cascade multiple devices with gains / losses G1,G2,…,GK and noise figures NF1,NF2,…,NFK, respectively, as illustrated in the figure below.

Noise figure with several cascaded devices.

Let G{1,…,k} denote the gain / loss and NF{1,…,k} denote the noise figure of the cascade of the first 1≤k≤K components, respectively.

Applying the result for the cascade of two devices recursively, we have

G{1,…,k}=i=1kGi

and

NF{1,…,K}=NF{1,…,K−1}+NFK−1G{1,…,K−1}=NF{1,…,K−2}+NFK−1−1G{1,…,K−2}+NFK−1G{1,…,K−1}

and so forth.

In particular, for K=3, we have

NF{1,…,3}=NF1+NF2−1G1+NF3−1G1⋅G2

## Adding a Low-Noise Amplifier (LNA)

The cascaded noise analysis above places significant importance on the first few devices having both low noise figure and high gain. It motiviates the addition of a device called a low-noise amplifier (LNA) to the front end of our radio receiver, as shown in the figure below.

Adding an LNA to the receive circuit.

As we will see in lab, it will make sense to add additional amplifiers in the receive chain to overcome the noise figure of the analog-to-digital converter as well.

## Refences

- H. T. Friis, “Noise Figures of Radio Receivers,” in Proceedings of the IRE, vol. 32, no. 7, pp. 419-422, July 1944. doi: 10.1109/JRPROC.1944.232049

- Henry W. Ott, Noise Reduction Techniques in Electronic Systems, 2nd Edition, John Wiley & Sons, 1988.