# Lecture 26 - Maximum Likelihood Estimation Framework

EE-30023/31023, Department of Electrical Engineering, University of Notre Dame

We have covered how to model many of the non-idealities present in real signals, but in practice these we can never perfectly model real signals. For this reason, we need some way to determine what data our imperfect received signal actually corresponds to. The framework that we will use to acheive this is maximum likelihood estimation.

## General Framework

Before delving into examples, we need to define a few terms. First, we have some parameter vector θ. We also have some probabilistic model fr|θ for how that parameter vector affects an output. In our case, the output will be r which is called our measurement or observation vector. Notice that the model is some conditional probability distribution, which means that the output is dependent on the input(s).

The maximum likelihood (ML) estimate θMLr is defined as follows:

θMLr=argmaxθ∈Φfr|θr|θ

For fixed θ, fr|θr|θ is a probability distribution. In principle it could be any type of probability distribution. For fixed r, fr|θr|θ is a likelihood function, i.e. the likelihood of that particular observed r as a function of different θ. When we maximize fr|θ, we find the most likely ELABORATE

## Jointly Gaussian Likelihood Function

For our purposes, the jointly gaussian distribution is important. Recall that it is completely specified by two quantities: - Mean vector Mθ - Covariance matrix Kθ

When we know the form of the mean vector and the covariance matrix, we can write the likelihood function like so, where n is the dimension of the vector r:

fr|θr|θ=12πn/2Kθ1/2exp−12r−μθT K−1θ r−μθ

Because we only care about maximizing this function, not its exact value, we can do some monotonic transformation on it to make the math easier. In this case, taking the logarithm will remove the exponential and make things simpler:

lnfr|θ(rθ)=−n2ln2π−12lnKθ|−12r−μθT K−1θ r−μθ

Very often, the covariance matrix doesn’t depend on parameter matrix. For instance, AWGN has a flat power spectrum so several of the terms simplify.

## Examples

### Unknown Mean, Known Variance

Suppose rk iid. Gaussian with mean θ and variance σ2.

We begin by writing our likelihood equation. Note that in this problem, θ is a scalar.

fr|θr|θ=k=1nfr|θrk|θ

Because we know the distribution is Gaussian, we can rewrite the equation.

fr|θr|θ=k=1n12πσ2e−rk−θ2/2σ2

If we now take the logarithm of this expression, we get that

lnfr|θr|θ=k=1n−12ln2πσ2−rk−θ22σ2

Now, for any given r, we want to find the θ that maximizes the function f. In other words, given the data that we observe, what is the parameter that makes the data most likely?

Recall the maximum likelihood function we defined above:

θMLr=argmaxθfr|θr|θ=argmaxθlnfr|θr|θ

Notice that in our likelihood equation, the first term does not contain θ. Notice also that there is a negative sign in front of the second term (which does contain θ). Maximizing a negative of some quantity is the same as minimizing the quantity, so our maximum likelihood equation becomes

θMLr=argminθk=1nrk−θ22σ2

To find the minimum, we can take a derivative with respect to our paramter and set it equal to 0.

ddθk=1nrk−θ22σ2θ=θML=0

which can be rewritten

k=1n−2rk−θML2σ2=0

We can simplify this expression further to

n θML=k=1nrk

which leaves us with an equation for θML:

θML=1nk=1nrk

What is this saying? The mean of the data should be the mean of the sampled data.

There are a few final things to note about this problem: - We call this estimator a sample mean estimator. - The result is independent of variance - EθMLr=1nk=1nErk=θ, which means this receiver is unbiased - The mean-square error of this estimator is EθMLr−θ2=1nr2, which approaches 0 as n→∞. We say that this makes the estimator consistent. - In general, the more data we have, the better our estimate will be with this receiver.

This is perhaps the simplest estimator we will cover, but it illustrates the concepts of maximum likelihood detection well.

### Known Signal in Noise of Known Variance, Unknown Amplitude

Suppose rk=θsk+wk where sk is known and wk N0,σ2 iid. We can write the likelihood function:

fr|θr|θ=k=1n12πσ2e−rk−θsk2/2σ2

We take the derivative and set it to 0

ddθfr|θr|θθ=θML=0

and end up with the following maximum likelihood equation

θMLr=k=1nrkskk=1nsk2

Recalling signal space concepts, if sk=⟨st,φkt⟩, rk=⟨rt,φkt⟩, and wk=⟨wt,φkt⟩ and we let n→∞ so that we have a complete orthonormal basis, then we can think of our maximum likelihood equation in terms of waveforms.

θMLrt=⟨rt,st⟩⟨st,st⟩

In integral form this would be

θMLrt=−∞∞rts*tdt−∞∞st2dt

We see here that we can solve more complicated continuous time problems with our vector math.