# Lecture 28 - ML Frequency and Phase Offset Estimation

EE-30023/31023, Department of Electrical Engineering, University of Notre Dame

## Examples

### Known Signal with Unknown Frequency Offset in Noise

rt=stej2πΔft+wt, where st is known, wt is AWGN, and Δf is unknown.

We will assume that st is bandlimited to max frequency B Hz, and that the frequency offset is in some set range (−f+max<Δf<fmax). We will sample rt with period T<12B+fmax. We will again use signal space concepts with the sampling basis (ideal unit sinc pulses in the time domain).

We can again write the each signal as a vector:

rk=⟨rt,φkt⟩

skΔf=⟨stej2πΔft,φkt⟩

wk=⟨wt,φkt⟩

Each has a corresponding vector form:

r=r1 r2 ... rnT

sΔf=s1Δf s2Δf ... snΔfT

w=w1 w2 ... wnT

We can now try to find the maximum likelihood estimate for Δf, ΔfMLr=argmaxΔffr|Δfr|Δf.

After several steps, we again arrive at a minimum distance problem:

ΔfMLr=argminΔfr−sΔf2=argminΔfr2−2Re{⟨r,sΔf}+sΔf2

Like last time, the first and last terms are independent of the parameter, leaving only the middle term for the minimization over Δf.

ΔfMLr=argmaxΔfRe{⟨r,sΔf}ΔfMLr=argmaxΔfRe−∞∞rts*tej2πΔftdt

This form is more interesting than we’ve seen before. After integration, the expression becomes a function of Δf, which can be thought of as a Fourier Transform!

The Fourier Transform.

This problem is very similar to the time delay example from last lecture. If we take the Fourier Transform of the signal term stej2πΔft, we end up with Sf−Δf. This form looks very similar to the st−θ form we saw yesterday. It makes sense, then that the block diagram for the detector looks very similar.

A discrete time implementation of the above might look like the following:

The Discrete Time Fourier Transform.

In practice, we often implement this detector using a grid search over evenly spaced frequencies. This means that we can use the FFT algorithm and choose the peak over those discrete frequencies. If there are more grid points to search over, the FFT will have to be larger. A larger FFT would increase the accuracy of the estimate at the cost of computation time.

### Carrier Frequency Offset Correction/Synchronization 31:00

If rnT=snTej2πΔfTn+wnT and we estimate that ΔfML, then we can apply the “correction”

rnTe−j2πΔfMLTn=snTej2πΔf−ΔfMLTn+wnT

where wt is still Gaussian noise. Ideally, this quantity would be equal to the original signal snT, i.e. Δf−ΔfML≃0. For large n, even a small value for this frequency offset can cause a large distortion.

In lab, we will get around the issue of large n by choosing a packet size that limits the distortion.

Packet example.

We use the preamble to estimate the frequency offset, and then correct that offset in the payload of the packet. Because the payload is not actual data that we would like to transmit, our data rate is necessarily limited to

LL+H

We can detect L bits of actual data for every L+H bits we receive.

### Joint Frequency Offset & Phase Offset

Suupose rt=stej2πΔft+Δφ+wt. Following some developments, we can write our maximum likelihood equation as follows:

Δf,Δφ=argmaxΔf,ΔφRe−∞∞rts*te−j2πΔf+Δφdt

In this case, we have two parameters to optimize over, Δf and Δφ. We are still taking a recieved signal, correlating against a model for what we think the transmitted signal and unknown effects will be.

Another way to solve the problem would be to perform the estimation in two steps: first, perform the optimization on one of the parameters, and then perform a second optimization on the other parameter using the result of the first.

### Joint Frequency, Phase, and Delay Offset

Suupose rt=st−τej2πΔft+Δφ+wt. Following the same developments as above, we can write our maximum likelihood equation as follows:

Δf,Δφ,τ=argmaxΔf,Δφ,τRe−∞∞rts*t−τe−j2πΔf+Δφdt

We follow the same pattern here: take the received signal, correlate against what the actual signal should look like with impairments, and then optimize.